# Glossary

Terms having an asterisk against them are general terminology references.

#### ACTION BAR

One of several bars which contain buttons. They enable model building and running without the need to use pull-down menus.

#### ACTIONS

Statements which may be executed when certain events occur. Actions can change the color or description of a part display, perform arithmetic calculations with attributes and variables, or provide logic needed for vehicle modeling.

#### ACTIVE ELEMENT

An Element that can push or pull a part is said to be active. The 'current' window on the screen is called the 'active' window.

#### ACTIVITY ELEMENT

The WITNESS Element used to model a physical activity, operation, process, or any activity that takes a finite time.

#### ADDBAR COMMAND

An actions language command used to add a bar to a histogram.

#### ADVANCED ACTION BAR

An action bar that allows you to perform advanced functions.

#### ALWAYS RULE

A track load or unload rule which forces every visiting vehicle to load or unload respectively.

#### ANTITHETIC

A form of random sampling in which the complement of the regular sample is used, consequently reversing any bias in the regular sample.

#### ARRIVAL

The independent introduction to the model of one or more new parts or fluids, e.g. parts or fluids that are not created by a machine or pulled from the WORLD by a machine, processor, conveyor etc.

#### ASSEMBLED LOCATION

A special location which may be used in an Output Rule to cause parts to be removed from the model and update the 'assembled' statistic.

#### ASSEMBLY MACHINE

Machine that has the ability to combine more than one part into one part.

#### ATTRIBUTE VALUE

Value which is tied to an individual part or labor unit.

#### AUTHOR

The writer of the simulation model. Used for display and reference purposes only.

#### AUTOSAVING AND AUTOSTEPPING

You can save a set of .sim files and then step forwards or backwards through them. This is useful for executing overnight runs (with real time intervals set) or for re-running stages of the model.

#### AVAILABLE TIME

The time a machine, pipe, tank or processor is available for work, including idle, busy, blocked, setup, repair and waiting for labor time.

#### BATCH ACTIVITY*

An activity in which more than one entity may be processed simultaneously.

#### BATCH MACHINE

A machine in which more than one part may be processed simultaneously.

#### BATCH MODE

Running of the simulation without displaying the animation.

#### BEGIN COMMAND

The command to reset the simulation to the its state it was in at when time = 0.0.

#### BLOCKED ELEMENT

The state of an Element which has finished an operation and is unproductive because the parts at the Element are unable to move on.

#### BREAKDOWN OF ELEMENT

The point at which an Element fails. The breakdown can be interactive (forced immediately, by using the interactive breakdown button on the standard toolbar) or non-interactive (set in the Element detail dialog).

#### BUFFER

A location to store a number of parts.

#### BUFFER RULE

This rule enables you to add or remove a dedicated input or output buffer for a machine.

BUSY STATE

The state of an Element that is doing productive work, for example, a conveyor that is transporting parts.

#### BUSY TIME

The time a machine spends in a busy or productive state, or the amount of time which must pass before a vehicle may enter a track once it has been vacated by another vehicle.

#### CALL COMMAND

The command by which demand for vehicles is generated.

#### CAPACITY OF ELEMENT

The maximum number of parts or vehicles that may be held by an Element, or the maximum quantity of fluid.

#### CHANGED LOCATION

A special location which may be used in an Output Rule to indicate that fluids have changed. WITNESS updates the changed statistics by the appropriate amount of fluid and (as the new fluid is counted as having entered the simulation) WITNESS also updates the entered statistics.

#### CLEAN

Cleans a continuous processing Element.

#### CLIPBOARD

The Windows clipboard is an area where cut or copied text is stored ready for pasting. All other data which can be cut or copied (for example, graphics) is stored on the WITNESS clipboard.

#### CLOCK DISPLAY

The screen display used to indicate the current value of simulated time.

#### COMMAND INPUT FILE

The means by which a sequence of commands may be issued without human intervention.

#### COMPOUND RULE

When a rule is specified within a rule, one is said to be COMPOUNDED with the other.

#### CONNECT RULE

An Input or Output Rule used to connect a pipe to another continuous processing Element.

#### CONSTANT VALUE

A fixed value.

#### CONTINUOUS DISTRIBUTION

A probability distribution in which any value within the specified bounds may be returned.

#### CONTINUOUS PROCESSING ELEMENT

WITNESS Element that can contain, process or flow fluid (that is, a pipe, tank or processor).

#### CONVEYOR

An Element which can move several parts in unison.

#### CO-ORDINATES

You can display the co-ordinates of the current cursor position in the window title bar. This is useful for determining the position that you want to place a video file, or for using the functions which access the screen co-ordinates of Elements (GETXPOSN, GETYPOSN, SETPOSN).

#### COUNT DISPLAY

Any display of a queue or line may be displayed as a set of objects or as a count of objects.

#### CREATE ELEMENT

The introduction into the model of a new part or fluid by a part arrival, or by an assembly, production or general machine.

#### CURRENT ENTITY*

The entity WITNESS is attempting to move, or has just moved, or the entity whose attribute values may be accessed.

#### CURRENT PART

The part WITNESS is attempting to move, or has just moved, or the part whose attribute values may be accessed.

#### CYCLE

The productive action of a machine, or the act of moving all parts on a conveyor forward one position.

#### CYCLE TIME

The time taken for a machine to complete one productive operation, or the time taken for a conveyor to index parts forward one position.

#### DEMAND ON ELEMENTS

Requests for vehicle or labor constitute the demand for those Elements. When a vehicle is traveling to respond to a demand, it is said to be demanded.

#### DESCRIPTION OF ELEMENT

The 1 to 4 characters which may be used to represent a part, labor unit or vehicle on the screen; the value of the DESC attribute.

#### DESTINATION RULE

This rule allows you to model tracks, which behave as points or switches, so that a vehicle can be directed to different tracks, according to its destination.

#### DESIGNER ELEMENT

A special Element from which other Elements can be created.

#### DETAIL COMMAND

The command to add modeling logic such as cycle times and breakdown data to the WITNESS Elements.

#### DID NOT ENTER*

Number of entities or items that did not enter the system upon arrival.

#### DIMENSION VARIABLE

Alternative term for quantity. Variables with a quantity of greater than one are said by programmers to be dimensioned. Variables can have a maximum of 16 dimensions.

#### DISCRETE DISTRIBUTION

A probability distribution in which only values specified are returned.

#### DISTRIBUTION ELEMENT

A WITNESS modeling Element that contains a series of numeric values and probabilities, or a WITNESS function, used to describe random behavior.

#### DOWN STATE

The state of a machine, pipe or processor that has broken down and has not been repaired.

#### DRAWBAR COMMAND

An actions language command used to record observations on a histogram.

#### DUMMY ELEMENT

An Element, such as a machine, that is used to control logic within the simulation and not used to represent a physical item.

#### DURATION OF OPERATION*

The time taken for an activity to complete one productive operation, or the time taken for a conveyor to index entities forward one position.

#### ELEMENT

A basic component of a WITNESS simulation model.

#### ELEMENT VARIABLE

The WITNESS variable giving the name of the current Element.

#### ELEMENT FLOW

Shows the movement of parts or fluids through the model as a series of lines (with arrows indicating direction). The ultimate destination of parts and fluids is shown at the end of the line (for example, SCRAP, SHIP). You can display rule names at appropriate points.

#### EMPTY STATE

The state of a buffer which contains no parts.

#### END OF FILE

When all the values in a file have been read, the model is said to have reached the end of file.

#### ENTITY DISPLAY SIZE*

The number of characters used to display an entity.

#### ENTITY ELEMENT*

The Elements which move through a model, representing work-in-progress, customers, transactions or things being modeled.

#### EVENT IN WITNESS

The mechanism by which WITNESS advances the simulation clock and changes the state of the simulation. For example, events occur when parts arrive in the system, when a machine completes an operation, when parts move forward one position on a conveyor, or labor becomes available to start a new job.

#### EXECUTE ACTION BAR

An action bar enabling you to run WITNESS in many of its modes.

#### EXPLODE COMMAND

The command that indicates the parts contained within modeling Elements, their current states and fluid contents.

#### EXPRESSION STATEMENT

An algebraic statement comprising one or more of a constant, variable, attribute or function linked by arithmetic operators such as plus or minus.

#### FED FROM and FED TO

Information on the rule editor or buffer detail dialog, indicating the source or destination of a machine, conveyor, buffer, part, pipe, tank, processor or fluid (that is, one which the machine or other Element, conveyor, buffer or part etc. does not control itself) if known.

#### FILE EXTENSION

One or more characters used to describe the type of a computer file. For example, WITNESS models are stored in files with extension .mod.

#### FIXED CONVEYOR

A conveyor on which all movement stops when the part at the front is unable to move off. A belt conveyor is an example.

#### FLOW RULE

An Input or Output Rule used to connect two continuous processing Elements.

#### FLUID ELEMENT

A fluid Element which is shown as a block of color within a pipe, tank or processor. (If there is more than one type of fluid, then each fluid is represented by a band of color).

#### FREE CAPACITY

The difference between the capacity of an Element and the number of parts currently in the Element, or the volume of fluid.

#### FRONT (CONVEYOR, BUFFER, TRACK)

The first position of a conveyor or a buffer, e.g. by default parts enter a buffer or conveyor at the rear and leave from the front. Also the position on a track where vehicles leave from.

#### FULL STATE

The state of a buffer in which the number of parts is equal to its capacity.

#### FUNCTION

A special word which may be used in an expression to obtain a value based on the status of the system, a random number, etc. You may create your own functions using actions.

#### GAP

A position on a conveyor, track or multi-station machine which is not occupied by a part or vehicle.

#### GENERAL ACTIVITY*

An activity performed upon a number of entities in order to produce a different number or the same number of entities.

#### GENERAL MACHINE

A machine which processes a number of parts to produce a different number or the same number of parts.

#### GROUP NUMBER

The reference number used to indicate a set of attributes.

#### GROUP OF ELEMENTS

One or more part types that share a common set of attributes. Where statistics for a quantity of Elements are amalgamated into one report, the reporting is said to be by GROUP.

#### HISTOGRAM

An Element used to record and display data in bar chart form.

I (VARIABLE)

A WITNESS variable which stores the index of the vehicle on the current track.

#### ICON

A symbol used to represent a machine, part, vehicle or other Element. An attribute of a part indicating the icon number used to represent the part on the screen. A miniature picture on action bar buttons.

#### IDLE STATE

The state of a machine or vehicle which has insufficient parts to perform its task.

#### IMMEDIATE ACTIONS

An action statement which is obeyed immediately.

#### INDEX

The reference number of a specific Element. This is a number from 1 to the quantity of that Element.

#### INDIVIDUAL ELEMENT

A unique entity (for example, a machine, or activity, with a particular index, the specific part in a buffer). When statistics on the Elements with a quantity are maintained separately for each index, the reporting is said to be individual.

#### INITIALIZATION ACTIONS

Actions that are executed at the start of a simulation run at time 0.0.

#### INPUT BUFFER

A dedicated buffer tied to a machine that is the only source of parts for that machine.

#### INPUT QUEUE*

A dedicated queue tied to an activity that is the only source of entities for that activity.

#### INPUT RULE

A set of instructions telling where and how an Element gets its supply of parts or fluids from the rest of the system.

#### INTEGER

A whole number, for example: 7, -15.

#### INTER-ARRIVAL TIME

The time between arrivals of lots (groups of parts that arrive at the same time) into the system.

#### ITEMS* (ELEMENTS)

Items are Elements shown as a block of color within a stream, store or process. (If there is more than one type of item, then each item type is represented by a band of color).

#### JOB

A task or assignment for one or more units of labor.

#### JOIN ACTIVITY*

An activity that has the ability to combine more than one entity into one entity.

#### LABOR

Scarce resources (either physical or human) which may be required in order that other Elements may complete a process, for example, to repair a machine. You can model different skill levels for the same labor unit by using labor attributes, or allocate a specific person or resource to a particular job by specifying the index of the labor unit in a labor rule.

#### LABOR ATTRIBUTES

Attributes that are assigned to specific labor instances. The attributes can be used in labor rules, using the MATCH rule. This means that you can model different skill levels for the same labor unit.

#### LABOR RULES

Rules that detail the use of labor Elements for various activities. You can set labor rules for: machines (setup, cycling and repair), conveyors (repair), pipes (repair, flowing, cleaning, purging) processors (repair, cleaning, filling, processing, emptying) and tanks (flowing, cleaning).

#### LEAST RULE

Input or Output Rule, using the capacity of Elements criteria to determine processing order of Elements on a route.

#### LENGTH OF PART

Physical length is compared with speed to determine the time required for a vehicle to travel along a track. Part length is the maximum number of parts which there is physical space for in an Element, e.g. the number of parts that may be simultaneously on a conveyor, assuming that maximum capacity is equal to part length.

#### LEVEL LIMIT

A specified limit which, when reached, an action is taken (for example, when a tank fills to a certain level and is in danger of overflowing, an action can be taken to empty the tank).

#### LIBRARY FILE

A file, which contains all model information in text form and which, may be used to generate a model.

#### LOAD

The process of moving parts onto an Element.

#### LOT (JOB)

A group of parts that arrive at the same time.

#### LOT SIZE

The number of parts that arrive simultaneously in the system.

#### M (VARIABLE)

A WITNESS variable set to the number of a part within a lot or batch or set to the number of a plot in a timeseries.

#### M.T.B.F.

Mean Time Between Failures. The average time between breakdowns this is often quoted by equipment manufacturers.

#### MACHINE ELEMENT

The WITNESS Element used to model a physical machine, operation, process, or any activity that takes a finite time.

#### METEOR TRAIL

The meteor trail button on the execute toolbar starts the meteor trail recording for the model. The meteor trail report can, for selected parts, show (either on the selected window or on the process view) the flows through the model in the time period (either by using different line widths or counts) and it can output the flow information to a file.

#### MATCH RULE

You may only use a MATCH rule for a machine’s Input Rule. The MATCH rule allows a machine to input matching sets of parts for processing.

#### MIXTURE INTERVAL

The time interval allowed between calculations to determine the proportions of each fluid within indeterminate mixtures.

#### MODEL NAME

The name used by WITNESS to refer to the description of the simulation model.

#### MONITOR INTERVAL

The time interval allowed between calculations to determine the value of continuous processing Element functions and time-based Input or Output Rules.

#### MOST RULE

Input or Output Rule using capacity criteria to determine processing order of Elements. Input or Output Rule, using the capacity of Elements to decide on a route.

#### MOVE RULE

This Input or Output Rule moves fluid instantaneously from one Element to another.

#### MULTI-CYCLE MACHINE

A multi-cycle machine is a special type of general machine and is used to simulate machines, which perform a number of separate cycles to complete an operation.

#### MULTI-DIMENSIONAL VARIABLE

A variable with a quantity greater than 1. You can specify up to 16 dimensions for each variable. For example, this is useful for defining counters of stock in three or four dimensional storage areas or to represent attributes of items grouped by several dimensions.

#### MULTI-STATION ACTIVITY*

A multi-station activity operates as a number of single activities linked together. The number of stations is variable and each entity moves through all stations on the activity.

#### MULTI-STATION MACHINE

A multi-station machine operates as a number of single machines linked together. The number of stations is variable and each part moves through all stations on the machine.

#### MULTI-TASK ACTIVITY*

A multi-task activity is a special type of general activity and is used to simulate activities that perform a number of separate tasks to complete an operation.

#### N (VARIABLE)

A WITNESS variable set to the index of the station WITNESS is currently processing.

#### NAME OF ELEMENT

Up to 8 characters, the first of which must be a letter, used to identify modeling Elements, or a variable or attribute which may only contain a valid Element name and optional index.

OLE

Object linking and embedding. A term used to describe a range of features including the ability of one application to control the behavior of a second application.

#### OPERATIONS

Cycles completed by a machine or processes completed by a processor.

#### OPERATOR (ARITHMETIC)

An algebraic manipulator such as plus or minus.

#### OUTPUT BUFFER

A buffer that is tied to one machine, and is the only destination for parts coming from the machine.

#### OUTPUT QUEUE*

A queue that is tied to one activity, and is the only destination for entities coming from the activity.

#### OUTPUT RULE

A set of instructions used to describe how parts, vehicles or fluids leave an Element.

#### PAN

Sometimes an Element may be out of view on the virtual screen. You can pan to a selected Element (or, if the quantity of the Element is greater than 1, you can pan to an indexed Element, or all Elements in that group) using the standard action bar. This centralizes it within the current window.

#### PARAMETER

A value specified when a function or command is invoked in order to completely specify the action required of that function or command.

#### PARK

The process by which a vehicle becomes dormant, ceasing to travel around the tracks in the model.

#### PART DISPLAY SIZE

The number of characters used to display a part.

#### PART ELEMENT

An Element which moves through a model, representing work-in-progress, customers, transactions and the like.

#### PASSIVE

The inability of a station to push or pull parts.

#### PEN ATTRIBUTE

A numeric representation of the color combination with which an item is displayed.

#### PERCENT RULE

The routing of a part may be probabilistic or random in some way, rather than always following the same pattern. The PERCENT rule enables a routing decision to be based on a probability. Enter a list of Elements and their percentages indicating the probability of each Element supplying or receiving the part or vehicle. If the percentages do not add up to 100%, WITNESS will automatically normalize them so that they do.

#### PIPE ELEMENT

A pipe conveys fluids from one location to another and is displayed by two lines, representing the size of the pipe.

#### PIXEL

Picture Element, the smallest indivisible point of color on a computer screen. A unit of measurement for display objects.

#### PRE-EMPTION

The act of interrupting the process of one Element in order to obtain labor for another Element. Pre-empt level is a property of machines, conveyors, tanks, pipes and processors which is used to decide whether this may take place or not.

#### PRINT SCREEN

WITNESS facility that allows you to produce a hard copy of your model display (currently active window).

#### PRIORITY OF ELEMENT

A property of machines, pipes, tanks, processors, conveyors or transportation demands which determines which Element takes precedence. Typically used to break ties between Elements where several are competing for parts, vehicles or labor (see also pre-empt level).

#### PROCESS* ELEMENT

An activity which inputs items, processes them for a specified time, then outputs them. Processes are represented by rectangles, icons or both.

#### PROCESS VIEW

A separate window which contains a view of the processes in the model. Elements are displayed as boxes, with lines and arrows indicating the flow of parts or fluids. You can display rule names at appropriate points. See also the meteor trail.

#### PROCESSOR ELEMENT

A machine which inputs fluids, processes them for a specified time, then outputs them. Processors are represented by rectangles, icons or both.

#### PRODUCTION MACHINE

A machine that can produce one or more new parts from an input part.

#### PROMPT (RULE OR ACTION)

A WITNESS service where the editor prompts the user for the input necessary to complete a rule or action statement.

#### PROMPTING ACTION BAR

An action bar which gives information about the functionality of menu options and buttons.

#### PSEUDO-RANDOM NUMBER (P.R.N.)

A computer-generated number exhibiting all the behavior of a truly random number but in a reproducible fashion. A P.R.N.G. is a pseudo-random number generator. A P.R.N.S. is a pseudo-random number stream.

#### PULL RULE

The movement of a part from one Element to another according to the rule held against the Element that inputs the part. More specifically, PULL is the simplest Input Rule to control this movement.

#### PURGE

Cleans a pipe by passing fluid through it.

#### PUSH RULE

The movement of a part from one Element to another according to the rule held against the Element that outputs the part. More specifically, PUSH is the simplest Output Rule to control this movement.

#### QUANTITY ELEMENT

The number of Elements that have identical properties. You can define quantities of up to 999 for all Elements except parts (which have no quantity defined), variables (which have a maximum quantity of 99,999) and attributes (which have a maximum quantity of 15,000).

#### QUEUE LOCATION*

A location to store a number of entities.

#### QUEUING CONVEYOR

A conveyor that continues to move parts when the part at the front of the conveyor cannot move off, causing parts to accumulate or queue.

#### RANDOM NUMBER

A number generated by WITNESS to cause random behavior.

#### RANDOM NUMBER STREAM

The reference number of a reproducible sequence of random numbers. Properly called a pseudo-random number stream.

#### REAL (QUANTITY)

A numeric quantity that can take on any decimal value, for example, 7.5

#### REAR (CONVEYOR, BUFFER, TRACK)

The default position at which parts join conveyors. The default position at which vehicles join tracks.

#### RECIPE RULE

Input or Output Rule used to get known volumes of fluid (or items) from specified sources or send known volumes to specified destinations.

#### RECORD COMMAND

An actions language command used to record observations on a histogram.

#### REFLECT OBJECT

The act of transforming an icon into its horizontal mirror image, whereby the extreme left pixels are swapped with the extreme right pixels, and so on.

#### REJECT (PART, FLUID)

A part or fluid that cannot enter the system when it arrives.

#### REPAIR ELEMENT

The point at which a broken down machine, conveyor, pipe or processor becomes fixed, or the act of fixing such an Element. The repair can be interactive (forced immediately, by using the interactive repair button on the standard toolbar) or non-interactive (set in the Element detail dialog).

#### REPLICATION

A single part of an experiment, which produces one set of results. A number of replications are run, and each replication's random number streams are initialized so that they are different. You can record the results for each replication at any point during the replication (for example, at the end of each one).

#### RESOURCES*

People, or equipment, which may be required in order that other Elements may complete a process, for example, to repair an activity. You can model different skill levels for the same resource by using labor attributes, or allocate a specific resource to a particular job by specifying the index of the resource in a labor rule.

#### ROUTE LOCATION

When an Element attempts to push to route, it uses the next destination in the ordered sequence of stations through which the part is directed in the part route.

#### ROTATE (OBJECT)

The act of turning an icon through 90 degrees in a clockwise direction.

#### RULE (WITNESS)

A set of instructions which controls the logic of movement of parts, fluids and vehicles between Elements or the allocation of labor.

#### SCRAP LOCATION

A special location which may be used in an Output Rule to cause parts to be removed from the model and update the 'scrapped' statistic. You may specify that parts are scrapped when the machine they are on breaks down.

#### SELECT RULE

This rule permits a part or vehicle to be routed based on some current condition of the simulation. WITNESS will evaluate the integer expression specified and pick a destination in the given list based on the current value of that expression.

#### SEQUENCE (PARTS)

A cyclical ordering of part flow.

#### SEQUENCE RULE

This rule is useful when routing needs to follow a cyclic SEQUENCE. WITNESS selects the first Element in the list until the specified number or quantity of items has been transferred. Then it selects the second Element in the list until its requirements have also been satisfied. Once the end of the list has been reached, WITNESS selects the first Element again. If a receiving Element is full (or a supplying Element is empty) the Element can WAIT for the next part or vehicle, use the NEXT Element in the sequence, or RESET the sequence to the first Element in the list.

#### SET ACTION

An action statement used to change an Element icon, quantity, quantity of labor, capacity, life or the amount of life before a machine setup or walk on and off modes.

#### SETUP ELEMENT

The state of a machine or other Element that is undergoing a tool change or planned maintenance. WITNESS can simulate multiple setups to represent tool changes, part changeover etc.

#### SHIFT ELEMENT

The shift Element allows you to represent working periods for labor and other Elements. Complex shift patterns can be built up using main shifts and sub-shifts. You can read shift pattern data into the model from a .sft file via the shift detail dialog, or by using the READSHFT function in an action statement.

#### SHIP LOCATION

A special location which may be used in an Output Rule to cause parts or fluid to be removed from the model and update the 'shipped' statistic.

#### SINGLE ACTIVITY*

An activity that processes one entity at a time.

#### SINGLE MACHINE

A machine that processes one part at a time.

#### SITUATION (EXPERIMENTATION)

A distinct configuration of the system being modeled, with the aim of using the experimentation to compare the effectiveness of different situations.

#### SKIP OBSERVATIONS

If the number of observations from a pseudo-random number stream should be discarded, these are said to be skipped.

#### SPACING (PARTS)

The distance specified as a number of characters between individual parts on the display.

#### SPEED (VEHICLE)

A property of vehicles used to determine the amount of time taken to travel along a track.

#### SPLIT ACTIVITY*

An activity that can produce one or more new entities from an input entity.

#### STANDARD ACTION BAR

An action bar which allows you to perform standard functions.

#### STATE OF ELEMENT

The current status of an Element at a given point in time. Each possible state has a reference number.

#### STATEMENT COMMAND

A complete action command.

#### STATUS OF ELEMENT

A machine, pipe, tank, processor or vehicle with a status display changes color as it changes state.

#### STEP COMMAND

The command to advance the model from one significant event to the next, one step at a time.

#### STOPPAGE* ELEMENT

The point at which an Element ceases normal operation.

#### STORE* (ITEMS)

A queue for items. Stores are represented by rectangles, icons or both.

#### STREAM* (ITEMS)

A stream conveys items from one location to another and is displayed by two lines, representing the size of the stream.

#### STRING (TEXT)

A collection of text characters.

#### STYLE ELEMENT

Display format, used to indicate how empty tracks, conveyors and other Elements should be represented on the screen.

#### SUB-MODEL

One or more Elements of a model saved as a sub-model, which can then be used in other models (or on more than one occasion in the same model). Sub-models have the filename extension .sub.

#### TANK ELEMENT

A buffer for fluids. Tanks are represented by rectangles, icons or both.

#### TIME VARIABLE

The WITNESS variable giving the current simulation clock time.

#### TIME-WEIGHTED AVERAGE

An average number that has been calculated with regard to the amount of time for which each value was observed.

#### TIMESERIES

An Element used to record data in the form of a graph. The timeseries can report on the maximum, mean and minimum plottings over a repeated time period.

#### TITLE OF MODEL

The long description of a model.

#### TRACK ELEMENT

An Element used to guide a vehicle.

#### TRANSPOSE DIMENSIONS

Two dimensional variables (or multi-dimensional variables, for which you may display two dimensions) may be displayed with the first dimension as columns and the second as rows; the rows and columns may be transposed to reverse this.

#### UNDEFINED VALUES

Values which have not been supplied. If WITNESS finds that an important value has been left undefined when the model is run, it will prompt you to supply a value or assume a value of zero.

#### UNLOAD ELEMENT

The act of transferring parts from an Element.

#### UPDATE LOCATION

Refresh display at same location using new information.

#### USER ACTIONS

A stand-alone set of actions invoked by the run/user actions command.

#### VALUE ELEMENT

Current value of a storage Element; an expression may be thought of as a 'potential value', to be converted to a value (evaluated) when required.

#### VARIABLE

An item or set of items of data. This can be referenced from any point in the model. The maximum quantity of a variable is 99,999 and they may be multi-dimensional (with up to 16 dimensions).

#### VEHICLE

When combined with tracks, this Element is used to model flexible material handling systems. Vehicles carry parts.

#### VIRTUAL SCREEN

The surface on which WITNESS Elements are displayed. It is typically very much larger than the physical screen, and portions of it may be viewed through windows. It is analogous to the worksheet of a spreadsheet program.

#### VISUAL RULES

Rules applied to Elements using a combination of buttons on the visual rules action bar and your mouse. They show, in your current window, graphical or textual portrayal of a rule being applied.

#### VOID FUNCTION

A function which cannot return a value.

#### VSEARCH COMMAND

The command which indicates the tracks from which vehicles may be drawn in order to satisfy a CALL (c.f. WORK SEARCH).

#### W.I.P.

Work In Progress. The number of parts (or amount of fluid), either of a particular type or all part types, currently in the system.

#### WAIT RULE

A null input and Output Rule, which causes a part or fluid to remain at its current location until acted upon by another Element.

#### WAITING ELEMENT

The state of a machine, conveyor, pipe, tank or processor that has parts to process and is unable to obtain labor.

#### WALK

A command which switches walk mode on or off. The command to cause a model to be run indefinitely with full animation being displayed, including displayed movement of parts.

#### WARMUP

The period at the start of a simulation run during which the model reaches a steady state and after which statistics collection begins in earnest.

#### WASTE FLUID

The fluid equivalent to scrap. Purge fluid goes to it automatically.

#### WEIGHT (IN DISTRIBUTION)

The relative frequency with which a value in a distribution has been observed.

#### WORK SEARCH

A list of tracks stored against a track which indicate which tracks may be searched for work when vehicles become idle on that track (see also VSEARCH).

#### WORLD LOCATION

The origin of parts and fluids.

#### ZONE LIMIT

A set of tracks within which there is a limit to the number of vehicles that may be present.

#### ZOOM (IN and OUT)

The act of magnifying or reducing the contents of a window.