Advanced Features

Objectives

- To introduce you to the concepts of Modules and Hierarchical Modeling.

- To build models effectively using Modules.

- To learn the advantages of using Designer Element or DES files.

- To handle Part files and Data files.

- To expand your knowledge on the use of actions and functions.

## Modules and Hierarchical Modeling

### Introduction

Most simulation projects involve building a model or models that will be presented not only to engineers and other technical personnel, but also to managers and executives as well. Personnel at many different levels of an organization may be involved in the decision making process to implement the results of a simulation study. Engineers will normally be more interested in the technical issues, but managers and executives will want to see the “big picture” and the “bottom line.” Therefore, simulation models should be built with individuals at all levels in mind.

### Modules

WITNESS has an Element type called Module that allows fast creation of hierarchical or 'black-box' modeling. A module can hold a collection of other Elements with input and output point. All the features of individual Elements are retained. When Element details are changed the Module will be updated accordingly. You can save modules as MDL files in order to use them again in other models.

You can use a module in several ways:

- You can detail the process in the module and concentrate on examining the elements in that particular process.

- You can detail the process in the module but then collapse the module down so that you can concentrate on the top-level model design. Elements pass from the model to an input point (or element) in the module, through the appropriate elements within the module, then back to the model again via an output point (or element) in the module.

- You can even enter an approximate cycle time for the module so that you can run the complete model and then fill in the elements in the module later. If the module uses a cycle time, you can also generate reports for the module.

- You can detail the process in a module, save it to a module file (*.mdl) which contains all the information about the module, its elements and icons and so on, then load the file into other WITNESS models.

- You can create modules within other modules. This is known as Hierarchical Modeling.

Figure 6-1: Hierarchical Modeling

A Module may be defined and displayed in a similar way to other Elements. The following characteristics for a module, where appropriate, can be displayed:

- Name

- Title

- Icon

In WITNESS there are basically two ways to create modules: A Net and a Tree Method. The first one allows you to automatically create a module, a module file (.mdl) and even a Designer Element from a set of pre-defined and displayed Elements. The Tree Method, on the other hand, provides you with a greater control and flexibility on what you are doing.

### Global Versus Local Element Names

In the following sections, we will be covering both the Net and Tree Method on how to create modules; however, in order to understand them, we need to understand the difference between Global and Local Elements Names in WITNESS.

A Global Element Name is the actual name of the element stored in WITNESS for a specific simulation model. On the other hand, a Local Element Name is a shortcut version of the global name that could be used within a specific module.

The format for Global Element Names is the following:

MdName.MdName….Name

Where,

| MdName | Stands for the Name of a Module |
| --- | --- |
| Name | is the Local Element Name |

#### Understanding the Difference between Global and Local Names

A model within a factory might consist of a large number of WITNESS elements. John Goodkid, who just attended a WITNESS training course, has decided to use hierarchical modeling to create this simulation model. While modeling the factory, he decides to create a module called CELL containing two additional modules: SAW_AREA and PAINT_AREA.  Each one of these last two modules contains a Machine, a Buffer, and a Conveyor. While in the Cell, parts move from the SAW_AREA to the PAINT AREA module.

Figure 6-2: Global versus Local Element Names

There are only a couple of things Goodkid needs to do to finish his model:

- He needs to push parts from the Machine to the Buffer within each module.

PUSH to BUFFER

- He also needs to push parts from the Conveyor in the SAW_AREA to the PAINT_AREA Machine.

PUSH to CELL.PAINT_AREA.MACHINE

- If Machine is defined as the input element for PAINT_AREA, PUSH to CELL.PAINT_AREA would be equivalent to PUSH to CELL.PAINT_AREA.MACHINE.

As you can see, any reference to (from) an element from (to) an element within the same module uses local names. However; as soon as Goodkid decides to point to or retrieve from an element outside the current module, Goodkid needs to specify the full name of the element (Global Element Name).

The name of a module could be up to 24 characters long for each module level and 21 characters for a Designer Element module.

### The “Net” Method

This is maybe the quickest way to create a module and at the same time the respective MDL file and Designer Element.

Creating a Module Using the Net Method

- Make sure you have created all the elements you want to include in the module.

- Capture all the elements that you want to include in the module in a net. - A net should be surrounding all your elements on the screen at this point. - Click on Create Module button on the Advance toolbar. - WITNESS prompts you to enter a valid name for the module.

- Type in the Module name in the Name input field. (You are limited to 21 characters including letters, numbers, and underscores). - WITNESS automatically generates a simulation module, inserts the elements contained in the net into this simulation module and saves the simulation module as an *.mdl file. It then creates a designer module that uses the same *.mdl filename. The only thing left for you to do is to define any display or detail information for this newly created designer module.

### The “Tree” Method

This method gives you more control and greater flexibility than the Net method when building a Module, a Module File and a Designer Element Module.

Creating a Module Using the Tree Method

- Choose ModelElements. OR Press the <F4> function key. - The Elements dialog box appears.

- Click the Define button. - The Define dialog box appears.

- Click on the “Simulation” option from the Parent field. - Notice Simulation is now highlighted in the Parent field. - If you are defining a new simulation element, you can create it in the main simulation element branch or in a module on a lower level. If you are defining a new designer element, you can only create it in the main designer element branch. Just click on the branch or module that you want to contain the element that you are creating.

- Type the Element name in the Name input field. - You are limited to 21 characters including letters, numbers, and underscores for designer elements and 24 for simulation elements.

- With your left mouse button click on the down arrow to the right of your Element Type list box. - The list opens. - Scroll down to select Module as the element type.

- Click the Create button. - Notice you can no longer modify the Simulation Element Type of the newly created Element.

- At this point, you could either select the Display and Detail characteristics of your module or wait until you have included the elements to be contained in this module.

- Click the Close button once to go back to the Elements dialog box and a second time to go back to your model

- Drag and Drop the elements you want to move into the newly created module. - If you want to copy the elements instead of moving them, keep the <Ctrl> key pressed down while dragging and dropping the elements into the new module. You could also clone an entire module.

- Close the Element dialog box.

### Creating a Module File and a Designer Element Module

Creating a module using the Net Method automatically creates a Module File and its respective Designer Element Module. On the other hand, if you use the Tree Method, you will need to create the MDL file and the designer element separately. In the previous section, we created the module with its respective “child” elements. The next series of steps would show you the steps to create an Module File and how to attached it to a designer element.

Creating a Module File (.MDL)

- Choose FileSave As. - The Save As dialog box appears.

- Select Module Files (*.sub) option from the List file of type pull-down list. - Enter the name of the file in the File Name field.

- Click OK. - The Save Module dialog box appears on the screen.

- Highlight the Module you want to save. - It is advisable to leave the Save Icons option checked if you want to re-use them in the new modules you will create based on this MDL file.

- Click OK. - If the Module File has being successfully saved, a confirmation message should pop-up on the screen. - Click OK.

Figure 6-3: Save Module Dialog Box

Module Files are a way of saving part of a model, such that the contents of this file may then be loaded into another model. They may be used to create libraries of commonly used model sections (e.g. shift patterns), or to combine 2 or more models into a single model. You can associate the file to a designer element.

To create a designer element module use the same steps (1-8) we described earlier for a simulation element module with only one small change: In step 3, replace the Simulation branch by the Designer Branch.

Once your Designer Element Module has being created, you need to associate a Module File (MDL) to this Element.

Associating a Module File to a Designer Element Module

- Double click the left button of your mouse on the designer element name of the module. - The Detail dialog box for the element should open. - The Designer Element dialog box for a Module is slightly different that the Simulation Element dialog box for a Module.

- Click on the BROWSE button. - Select the Module File you have already created. - You can permanently link a Module to a Module File by checking the Link To File option. If you do so some of the detail options of the resulting module will not be available to you. - Click OK.

Figure 6-4: Detail Dialog Box for a Designer Element Module

### Creating a Module from a Designer Element

Whether you use the Net or the Tree Method to create a Designer Element Module, the next step is to use this Designer Element to generate Simulation Modules.

Using Designer Elements to Create Simulation Modules

- Click the left button of your mouse on the Designer Element name of the Module. - A cross arrow appears

- Position the cross arrow where you wish the Module to be placed in the Simulation window.

- Click with the left mouse button. - WITNESS generates the name of the new simulation module automatically and displays the module display items on the modeling window. If the name of the Module File (MDL) file is Cell, then the name of the first module becomes Cell01, Cell02 and so on. - Then, WITNESS opens the Element Placement window you had selected in the Designer Element Detail dialog (Window 8 is the default).

- Position the cross arrow where you wish the Elements contained in the Module to be placed in the Element Placement window.

- Click with the left mouse button. - WITNESS automatically sets up an Associated View (to display the elements of the module) from the module display. - The Element Placement window is automatically closed. - At this point you are done unless you have selected the Multiple Designer Elements option from ModelOptionDisplay menu. If this is the case, you will also need to click the right button of your mouse once you are done creating modules.

Figure 6-5: Generating Modules from Designer Elements

It is important to mention that if you do not want to take the Element Placement window defined in the Designer Element Detail dialog, you can click on the ViewWindows option and select another window before going to step 5 on the previous set of instructions. However, WITNESS still creates an Associated View for the window defined in the Detail dialog box; therefore, you will need to update the Associated View of the new Module separately.

- Remember that while the  button stores an Associated View, the  button opens it.

A Module File contains a module with all its elements and associated data, including icons. When loading a Module File, the actual library icons are used only if the icons in the MDL file are identical to existing icons in the current library. If the icons in the Module File do not exist in the current library, they are added to the current library. If there is no room, WITNESS asks you either to select an icon in the library that can be overwritten, or to stop loading new icons.

WITNESS treats all attributes in an MDL file as global elements when they are loaded into the model. WITNESS prompts you to resolve any name clashes.

If you save a module that contains an element (A) that references another element (B), which exists in the main model but not in the module, WITNESS saves element B’s name and type to the MDL file. When you load the module file into a model, WITNESS checks whether element B’s name and type exist at the global (model) level. If not, WITNESS displays the Remap {element} dialog so that you can map element B to a different element, or skip references to element B.

Figure 6-6: Remap Element Dialog Box

| Table 6-1: Options for the Remap Elements Dialog |  |
| --- | --- |
| Item | Description |
| Element | Selects an element to map the element (B) to. |
| Remap | Maps element (B) to the selected element. |
| Skip | Closes the Remap {element} Dialog without mapping the element (B). WITNESS deletes the current rule, action or expression that is being remapped. |
| Skip All | Closes the Remap {element} Dialog without mapping the element (B). WITNESS deletes any rules, actions or expressions that reference element B. |
| Define | Allows you to create a new Element that could be used in the mapping process. |

### Detail Options for a Simulation Module

Once your Simulation modules have being created, you can modify different detail options depending on your needs and preferences. The Detail dialog box of a Module is divided into five tags or pages: General, Input / Output Elements, Actions, Reporting and Notes. The first three tags are covered in the following sections while the last two are not since they are similar to those of previous WITNESS elements.

#### The General Page

The General Page provides you with not only the options you need to determine how a module should run within a model, but also to determine how parts entering and exiting a module should be displayed on the screen.

Figure 6-7: Simulation Module General Page

| Table 6-2: General Page Options for Simulation Modules |  |  |
| --- | --- | --- |
| Item |  | Description |
| Name |  | The name of the module is unique. |
| Title |  | An optional Title can be assigned to a module for display purposes. |
| Module Run Mode |  | Usage of a module during the execution of a model. |
|  | Elements (Normal behavior) | Part that enters the module passes through all the appropriate elements and then leaves the module |
|  | Cycle Time (delay buffer) | Part enters the module but does not goes through the elements in the module. It waits for the specified cycle time to elapse before it attempts to leave the module |
| Link To File |  | Option is only enabled when you created the simulation module from a designer module and the link to file box on the designer module dialog was checked. |
| Password Protection |  | Protects the module from unauthorized changes. |
| Walk to Module |  | Displays parts walking to the module before moving to the elements within the module. It only applies when the walk-run mode is on. |

The Cycle Time option for a module is useful for speeding up the run-time of a complicated model if you know approximately how much time it takes to complete the processes in the module. It is also useful if you are using a top-down design approach for building your model (Hierarchical Modeling). You can create a module with an appropriate cycle time, then come back later and fill the module with the necessary elements.

#### The Input/Output Elements Page

This page gives you the possibility to assign the Elements as the input and the output points of the module and to apply rules to these Elements.

Figure 6-8: Input/Output Elements Page

| Table 6-3: Input/Output Elements Page Options |  |
| --- | --- |
| Item | Description |
| Element | Selects the element that is the input (or output) point for the module from the drop list. |
| Member | If the quantity of the input (or output) element is greater than 1, select the individual member element from the drop list, or select all instances of that element. |
| From…/To… | Allows you to enter an input rule that will be applied to the module’s input element and an output rule that will be applied to the module’s output element respectively. |

#### The Actions Page

Actions are the means by which you can interact with your model. The purpose of the Actions page is to allow you to interact with your modules in different ways to increase the usability and flexibility of your modules within models.

Figure 6-9: Actions Page for a Simulation Module

| Table 6-4: Actions Page Options for a Simulation Module |  |
| --- | --- |
| Item | Description |
| Initialize actions | Enter the action statements you want WITNESS to execute for the module each time you start the model run. |
| User Actions | Define the action statements you want WITNESS to trigger for any elements in the model (including the module) whenever you select the RunUser Actions command. |
| Properties actions | Enter the action statements that apply specifically to the module. If the module is linked to a file, you cannot set up this option. Run these actions by clicking on the Run Properties button. |

### Display tips for Hierarchical Models

WITNESS enables the user to build hierarchical display capability into any WITNESS model using layers and windows. A basic method of accomplishing this is described below:

- Identify the levels of detail you want to display (detailed, macro, etc).

- Select and rename desired layers to indicate what will be displayed on them.

- Build detailed simulation model with display of all Elements on the appropriate detail layer. This would be the display of the individual parts, buffers, machines, and other Elements in the model perhaps representing a particular work cell in a factory.

- Using the Screen Editor or Element display dialog for a particular Element, add the required graphics and text for the first macro level on the appropriate layer. This might be a block diagram of the work cells in the factory with pertinent work cell-level statistics displayed.

- Repeat the above step until all desired levels of display detail have been added to the model on separate layers.

- Set up dedicated view windows to show all of the levels of display. Rename each of the windows, set background color and zoom factor, and set only the required layer(s) to appear in each window.

- Associate the view windows with different model Elements to make them easier to display. Add text associated with these Elements describing what is displayed in each window that can be activated by clicking first on the text, then on the  button on the Standard Action Bar.

### Using Designer Elements Files (DES)

Sometimes after start a new simulation project, you realize that some parts of the model you need to work on have already being created for other models. For example, different testing and printing areas might have already being created as Designer Elements by one of GoodKid’s co-workers, BrightKid. BrightKid’s model has a Designer Elements template that has been tested and personalized to fit various situations in the plant. This template not only has customized machines or single elements but also modules. Therefore, Goodkid would love to be able to put his hands on BrightKid’s template. However, he has already started his model and he does not want to start over.

In WITNESS, you can create Designer Files (DES) or templates. These templates could contain up to 40 elements that can be saved and loaded in seconds. This capability allows WITNESS to have no limit on the number of designer elements that can be used in a model. Therefore, there are only a couple of steps that Goodkid would need to follow in order to incorporate Bright Kid’s Designer Elements into his model.

Creating a Designer Elements File (.DES)

- Make sure the model containing the Designer Elements you want to save is open.

- Choose FileSave As. - The Save As dialog box appears.

- Select Designer Elements Files (*.des) option from the List file of type pull-down list. - Enter the name of the file in the File Name field.

- Click OK. - The Designer Elements File is saved.

Opening a Designer Elements File (.DES)

- Make sure the model you want to contain the new Designer Elements File is open.

- Choose FileOpen. - The Open dialog box appears.

- Select Designer Elements Files (*.des) option from the List file of type pull-down list. - Select the file in the File Name field to be open.

- Click OK. - The Designer Elements File is opened into the current model.

### Last Comments on Hierarchical Modeling

Good planning could determine the success of a project. Hierarchical Modeling can be a very powerful tool. If used correctly, it might save hours of modeling. In this section, we have provided you with step-by-step instructions and descriptions on the generation of Modules, Module Files, Designer Element Modules and Designer Elements files so that you can successfully implement these tools into your models.

- Do exercises 1 and 2 now.

## Part or Link Files

A Part or Link File is a file on your computer’s disk that contains a list of Parts. For each Part or Entity in the file, you may specify the simulation time at which it will arrive in the model. This is useful for modeling simple production scheduling problems where the normal Part or Entity inter-arrival mechanism does not allow you to specify Part or Entity arrivals with sufficient precision.

Parts may also be output from your model to Part or Link Files; in this way it is possible to produce a Part File as an output from one model which is then used as an input to another model. This facility is also useful to trace the exact time at which Parts left the simulation and the value of their Attributes at that time.

The WITNESS part or link file Element creates an interface between the external file and WITNESS.

Creating a Part or Link File

- From the Designer Elements Windows, click with the left mouse button on the Element name Partfile. - A cross arrow appears.

- Position the cross arrow where you wish the part file to be positioned in the simulation window.

- Click with the left mouse button. - Partfile001 appears on the screen with a net around it

- Click and drag inside this net to move Pfile001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element.

- Select the detail icon from the Standard Action Bar. - The part file detail dialog appears on the screen.

- Specify the Actual Filename (this name does not have to be the same as the WITNESS filename, but it will default to this).

- Specify the format of the time in the file: Absolute – the time in the part or link file is the actual times the parts arrive. Relative – the time in the part or link file is the time between arrivals.

- Specify a Start Offset (if required) – the first time in the file will be incremented by the value specified.

- Specify a Delay (if required) – if the file restarts at the end, the delay can create a gap between the finishing time and restart time.

- Specify an Output Rule that specifies where the parts entering the model from the file will be placed when they arrive in the model.

Figure 6-10: Part/Link File Detail Dialog Box

### Example of Using a Read Part File

Read part files are used to create a schedule for the arrival of parts. The example below looks at creating an arrival schedule for planes at an airport.

The planes at Redditch International Airport have a specific arrival schedule. They arrive at: 8:00am, 10:00am, 2:00pm, 3:30pm, 6:00pm and 9:00pm. Obviously the current inter-arrival pattern for parts is not flexible enough to deal with these specific arrival pattern; therefore, we need to create a part file.

| Using a Read Part File |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Put the Part file in model. - Choose the PartFile Element from the Designer Elements Window and place in model. You can rename the Part File (default name e.g. PARTFILE001) to something more meaningful using the Detail form for the Part File; e.g., SCHEDULE. |  |  |  |  |  |
| - Create the external file containing the arrival schedule. - This file can be created in any external package, but you have to ensure the file is saved as a text file. (Avoid using Microsoft® Write™, as this inserts additional formatting codes) - Part Files have a specific format: |  |  |  |  |  |
|  | !Part Name Plane Plane Plane Plane Plane Plane | Lot Size 1 1 1 1 1 1 | Time 480 600 840 930 1080 1260 | Attribute Values Nopax =100 Nopax =200 Nopax =120 Nopax =90 Nopax =120 Nopax =75 |  |
| - When you save the file, it is suggested that you save it into the same directory in which your model is saved. The default file extension is .PAR. |  |  |  |  |  |
| - Return to WITNESS and update the detail of the Part File |  |  |  |  |  |

### Write Part File

In order to write to a Part File, you go through a similar process to a Read Part file:

- Define Part or Link File (e.g. OUT)

- Instead of an Output Rule of PUSH to SHIP you type

PUSH to OUT

A file called OUT.PAR is created which records the part type, the time at which it left the model and the value of the attributes for that part.

The resulting Part File can be inspected by minimizing WITNESS and running a text editor. Note that if you wish to inspect the file, you must first close the file from WITNESS by selecting BEGIN from the RUN menu or by closing WITNESS.

- Do exercise 3 now.

## Arrival Profiles

In general Parts can arrive into a WITNESS model in one of four possible fashions:

- Parts can be puled into the model by an element at any point in time as long as the part has being defined (Passive Parts).

- Parts can have an actual inter-arrival time. This could be a constant, an expression or even a distribution (Active Parts).

- Parts could be generated by an external file (Part Files)

- Parts could arrive based on a repeating pattern of arrivals for a certain type of part (Active Parts with Profile).

The first three points we just mentioned were previously discussed in this manual. The Active with Profile option is the only one left to discuss. This option is actually triggered from the detail dialog box of a Part. Arrival Profiles allow you to customize arrival patterns and is primarily aimed to customers in the service sector: For example:

- Complex paperwork generation profiles

- The arrival of people at hospitals, banks, supermarkets, and so on

- Maintenance work profiles

- Occurrences of crimes or emergencies

If you model a car inspection center, you could model an average of 200 customers during a 12 hour-day. However, in real life you might find out that most of the customer visits follow some type of cycle. Most of the visits take place from 6 to 8 AM, from noon to 2 PM and from 5 to 6 PM. This could be easily explained because people have to go to work; therefore, the perfect time to get their car inspected is just before work, during lunchtime and right after work. The other customer could evenly arrive throughout the day. This is a perfect fit for the Arrive with Profile option.

Figure 6-11: Arrival with Profile Option

Notice that whenever you select the Arrival with Profile option from the Detail dialog box of a Part, a new tag becomes available. The Arrival Profile page on the part detail dialog is used to define the part arrival profile.

| Table 6-5: Options in Arrival Profile Page |  |
| --- | --- |
| Item | Description |
| A Grid | Displays the time periods and volume of parts arriving for each time period. |
| Time Display | Allows you to select a time scale for the arrival profile. |
| Multiplier | Allows you to automatically change the volume of parts arriving for each time period. |
| Random Number Stream | Is used to distribute the arrival of parts randomly. |
| Smoothing | Allows you to modify the distribution of the arrival of parts. |
| Cumulative Time | Displays or hides the Cumulative Time column on the grid. |
| Import | Allows you to import data from an external text file into the grid. |

- For additional information on these options, look at the on-line help.

Figure 6-12: Arrival Profile Page

## Files

WITNESS provides the facility for using data files within your model. You may input values from a file (a READ File), and you may output values (a WRITE File). File input and output has many uses, for example:

- Specifying values to be used in the details of Elements, for example, the cycle time duration of Machines and Activities

- Producing reports tailored to your requirements, with your own layout and contents.

Like all other WITNESS Elements, Files must be defined. Note that you must define your Files with the correct type; you may only READ from a READ File, and you may only WRITE to a WRITE File. The values to be read should be valid constants of one of the four WITNESS data types; namely integer, real, name, or string. These files are text files, which you may edit using a word processor or editor, and print on your printer. The default extension for the file is “.DAT”.

### Writing to a File

The WRITE action command is used to write data to a file. The values to be written should be either integer, real, name or string. Every WRITE statement will create a new line in the file. When using the WRITE command a number of special characters may be used to enhance the format of the file.

#### Usage

WRITE <filename> <list of variables/attributes>

#### Examples

WRITE FILE1 VAR1, VAR2, ATTRIB1

WRITE FILE1 “\f”

| \f | Introduces a form feed (e.g., clears the Interact Box or creates a new page in a file). |
| --- | --- |

WRITE FILE1 “\n”

| \n | Introduces a new line. |
| --- | --- |

WRITE FILE1 “\t”

| \t | Introduces a tab spacing. |
| --- | --- |

Create a File to Write Information Out to

- From the Designer Elements Windows, click with the left mouse button on the Element name FileWrite. - A cross arrow appears.

- Position the cross arrow where you wish the File to be positioned in the simulation window.

- Click with the left mouse button. - FileWrite001 appears on the screen with a net around it.

- Click and drag inside this net to move FileWrite001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element.

- To change the name of the Element or the actual filename of the file created, select the detail icon from the Standard Action Bar. - The File Detail dialog appears on the screen. (See Figure 6-13)

- In an appropriate action statement, e.g. Actions on the Start or Finish for a Machine cycle, type in WRITE FileW001 expression, where the expression is the information you wish to write out to file.

Figure 6-13: Write File Detail Dialog Box

The ‘WITNESS file name’ is used to reference the File within WITNESS, e.g., in READ and WRITE statements. The ‘Actual file name’ references the file on the hard or floppy disk. This does not have to take the same name as the WITNESS file name. It is also possible to specify the directory in which the file is held.

Actual file name: C:\PROJECT\FILE10.DAT

This facility is useful when testing alternative sets of data. The data could be held in a number of data files and the actual file name changed for each experiment to reference the correct File.

### Reading from a File

The READ action command is used to read data from a file. This may be used in any action statement, e.g. Actions on Initialize or Actions on the Start for a Machine cycle.

Entries may be separated by:

- Any number of spaces

- A comma (,)

- A semicolon (;)

- Being placed on separate lines

Every time WITNESS encounters a READ statement, it will read a new line of the file. However, if a semicolon (;) is placed at the end of the READ statement the next READ will start where the last READ finished. If a READ attempts to read more values than exist in the file, an error will occur. Data from a file may be read into variables or attributes.

The format of a READ statement is:

#### Usage

READ <filename> <list of variables/attributes>

#### Example

READ FILE1 VAR1, VAR2, ATTRIB1

Reading from a File

- From the Designer Elements Windows, click with the left mouse button on the Element name FileRead. - A cross arrow appears.

- Position the cross arrow where you wish the part file to be positioned in the simulation window.

- Click with the left mouse button. - FileRead001 appears on the screen with a net around it.

- Click and drag inside this net to move FileR001 to the required position. - If you have selected the Multiple Designer Elements option from the ModelOptionDisplay menu, you will also need to click the right button of your mouse before dragging the newly created Element.

- To change the name of the Element or the actual filename of the file to be read from, select the detail icon from the Standard Action Bar. - The File detail dialog appears on the screen. (See Figure 6-14)

- In an appropriate action statement, e.g. Actions on Initialize or Actions on the Start for a Machine cycle type in READ FileRead001 Var1, Var2. - Note that the variables Var1 and Var2 need to be used to store the data read in from the file.

Figure 6-14: Read File Detail Dialog Box

- Do exercise 4-6 now.

## Further Use of Actions

### Commands and Constructs

Each ‘action’ is a statement giving WITNESS a task to perform. Unless you give instructions to alter the flow of the actions, these statements are executed in a straight sequence, starting with the first and ending with the last. The following section lists a number of powerful extra facilities to give more control in the actions.

| Table 6-6: Action Control Commands |  |  |
| --- | --- | --- |
| Actions |  | Description |
| Part/Entity Control |  |  |
|  | CHANGE | Change name of Part or Entity |
| Interact Box Control |  |  |
|  | PRINT | Display message in Interact box |
|  | INPUT | Obtain input from user via Interact box |
|  | OPENBOX | Open Interact box |
|  | CLOSEBOX | Close Interact box |
|  | INTERACT COLOR CLEAR CURSOR | Sets color of text in Interact Box. Clears Interact Box. Area within box is black. Positions the cursor in the Interact Box.. |
| Data File Control |  |  |
|  | WRITE | Write to file |
|  | READ | Read from file |
|  | REWIND | Rewind file |
| Logic Control |  |  |
|  | IF/ELSEIF | Execute different statements depending on |
|  | ELSE/ENDIF | Value of logical expression or expressions |
|  | GOTO/LABEL | Continue Actions on specified line |
|  | FOR/NEXT | Execute statements a fixed number of times |
|  | WHILE/ENDWHILE | Execute statements while condition is true |
|  | RETURN | Exits from Actions, returning a value in the case of WITNESS functions. |
|  | STOP | Cause the simulation to stop running. |
|  | SET | Change a characteristic of a specified simulation Element(e.g. ICON, LIFE, QUANTITY, CAPACITY) |
| Statistical Recording |  |  |
|  | RECORD | Record a value in a Histogram |
|  | DRAWBAR | Draws a bar of a specified height and color in a Histogram. |
|  | ADDBAR | Adds a bar of a specified increment and color in a Histogram. |
| Comments |  |  |
|  | ! | Add remark to explain an action (may also be used in Input/Output Rules, Labor Rules, and at the end of an expression). |

### Special Strings

These may be used with the WRITE, PRINT, and INPUT commands. Examples are:

#### Examples

PRINT "\f"

| \f | Introduces a form feed Clears the Interact Box or creates a new page in a file. |
| --- | --- |

WRITE FILE1 "\n"

| \n | Introduces a new line. |
| --- | --- |

PRINT "\t"

| \t | Introduces a tab spacing |
| --- | --- |

### User Actions

There are many possible uses for User Actions; you may output reports to a File or prompt the user for the value of a critical Variable. It is even possible to create a dialogue that allows the user to largely bypass the regular WITNESS menus in order to experiment using the model.

To set up these User Actions, pull down the Model menu, click on User Actions and enter the appropriate set of Actions. These actions may then be run at any time by stopping the simulation run, pulling down the Run menu and clicking on User Actions.

### Immediate Actions

At any time a single line of actions may be executed by pulling down the Run menu and clicking on Immediate Actions. You are then prompted to enter a single action statement that will be executed immediately. This facility is very convenient. Examples of its use include.

- Printing the value of a Variable in the interact box

- Setting the value of a Variable

- Printing the value returned from a status Function, for example, to determine the current state of an Element

- Issuing a CALL for a Vehicle.

- Rewinding a data file

- Do exercises 7 and 8 now.

## Example

### Closed Loop Conveyor and Data Manipulation

#### System Description

The diagram shows a proposed closed loop conveyor system. The system operates as follows:

- Parts are enter the system at OP1_LOAD

- Operators place one part on an empty pallet at OP2. No empty pallet can pass OP2

- The pallet moves to the next operation via a conveyor.

- There are 12 unique operations each operation is separated by at least one section of conveyor.

- Each operation has its own unique cycle time and downtime characteristics. Additionally, each operation generates its own percentage of parts that need to be reworked.

- Pallets can not pass each other on any section of conveyor.

- Parts that need to be reworked enter the rework area at a spur that is located just after OP9 (tester). OP10 is the operation where the rework is performed. After the part has been reworked, the pallet reenters the system just before OP9 (tester).

- If a part does not need to go to rework the part proceeds to OP11 where it is taken off the pallet. The part proceeds to OP12 where it is outside the scope of the model. The empty pallets recirrculate through the system to OP2 where it will get loaded with a new part.

Figure 6-15: System Description

#### Objectives

Typical objectives for such a model may include the following:

- Can production requirements be met?

- What is the minimum number of pallets needed and still meet a certain production rate (e.g. jobs per hour)?

- How does improving our rework rates effect our production?

- How significant is the conveyor speed? What is the lowest speed that the conveyors can travel at and still meet production?

- Where are the bottlenecks? How much will production be improved if parallel operations are added at the bottlenecks?

#### Model Construction

Getting data into the model
Since this model is of a proposed system, parameters such as cycle times and breakdowns have a higher degree of uncertainty. This usually means more experimentation occurs because of various sensitivity and what-if analysis that need to be done. It is important to construct the model in such a fashion so that experimentation can easily be done. To do this it is good practice to separate the data from the model.

Separating the data from the model is usually performed in the following manner:

Figure 6-16: Close Loop Conveyor Example

- Create a spreadsheet (see above) that contains the system parameters that will likely be changed during experimentation.

- Structure the spreadsheet in such a format so the data can easily be saved as a text file. The intent is to take the data from the spreadsheet convert it into a text file and have WITNESS read it into the model.

Figure 6-17: Converting Spreadsheet Data

The major spreadsheet packages have a Save As feature in which the data in the worksheet can be saved as a text file (e.g. .csv, .txt)

The WITNESS model needs to be constructed in a format so that the data can be read from the text file and put in the appropriate place in the model. This is done by creating variables and using these variables for cycle times, breakdowns times, and rework percentages. The data from the text files is read into the variables.

The following is an example of reading in values into WITNESS.

Initialize Actions:

! set the number of pallets in model

READ infile NOPALL

! set the cycle times for the operations, breakdowns, scrap rates

!!!!!!!!!!    OP1_LOAD    !!!!!!!!!!!!!

FOR x = 1 TO 12

READ infile cyc (x,1),cyc (x,2),cyc (x,3),mtbf (x),mttr (x),ScrapOp (x)

NEXT

! set the delay times and capacities for the conveyors

! delay (x) = inches / (FPM*12)

! xx is dummy variable

FOR x = 1 TO 16

READ infile xx,xx,xx,xx,xx,xx,DELAY (x),mqueue (x)

NEXT !

!

Separating the data from the model has additional benefits besides making it easier to do experimentation; it also helps when it comes to validating the model. Since most of the system parameters are contained in one location, it makes it easier to spot unrealistic cycle times or breakdown parameters.

#### Modeling Techniques

A Part is used to represent a pallet.

Delay Buffers where used to represent the various sections of Conveyors as opposed to using the Conveyor Element. This was done to increase model speed. Conveyor Elements create more events (when they index) than a delay buffer, the more events in the model slower it runs.

The delay Buffers could be used because the following conditions where met: 1) the Conveyors where queuing; 2) the cycle times of the operations at the front of the Conveyors took a significantly longer time than the index time of the Conveyor.

If condition (2) was not met and the cycle time of the operations where shorter than the index time of the conveyor the system would not be correctly model. The problem occurs if there is queuing on a section conveyor before an operation. When a part has spent its expected time in the queue, it is free to leave. Example: There are two parts waiting on a conveyor for an operation to complete. If the operation takes the first part and processes quicker than the expected time for the second part to move to the front of the conveyor the operation would be able to get the second part sooner than what it could in reality.

The time the part spent in the delay buffer (conveyor section) was calculated as follows: conveyor length / conveyor speed.

A user-defined function where station number was passed is used in the downtime section for MTBF and MTTR. The advantage of using a user-defined function in which the parameters specific for the operations are passed is it is easier to change the distribution representing the downtimes. The distribution for all operations can be changed at one location the user defined function.

An attribute is used to keep track whether a part needs to be reworked.

## Exercises

### Exercise 1

#### Sub Models

The aim of this exercise is to use the modules feature within WITNESS to join together two of the models developed on the course; i.e. FINAL.MOD (from Exercise 1 in Chapter 5) and ROUTE.MOD (from Exercise 12 in Chapter 3).

Save FINAL.MOD as a module and join it to ROUTE.MOD so that the parts A, B and C produced by the job shop model (ROUTE.MOD) are fed into the buffers B1, B2 and B3 respectively.

- Save this model as JOINED.MOD.

### Exercise 2

#### Modules

The aim of this exercise is to use the module feature within WITNESS. Save the OP20 machines and CON1 conveyor as a module. Add a Designer Elements building block for the module and get it to read in this model. This enables you to store a group of elements as a module.

Add the module to the model from the Designer Elements building blocks window and join to model (e.g. at end of model) using the input and output rules from the module.

- Save this model as MODULES.MOD.

### Exercise 3

#### Input Schedule for Parts

Return to the model FINAL.MOD for this exercise.

Whereas previously parts arrived with regular inter arrival times, the parts A, B and C now arrive at the buffers B1, B2 and B3 respectively according to the schedule in the table below.

| Table 6-7: Exercise 3 Data |  |  |
| --- | --- | --- |
| Part Type | Batch Size | Inter Arrival Time |
| A | 2 | 0.0 |
| B | 3 | 20.0 |
| C | 2 | 20.0 |
| A | 3 | 20.0 |
| C | 3 | 20.0 |
| B | 2 | 20.0 |

Having been completed, the schedule is repeated after a 20-minute delay. Include this information in your model.

### Exercise 4

#### Writing Data to Files

A management report has been requested, indicating the output rate of parts from the first basic model. The report is to contain information relating to the type of part produced and the time at which that part was dispatched from the model. The management report should follow the company's standard format as follows.

Figure 6-18: Simulation Study Group

### Exercise 5

#### Reading Data from Files - Part 1

The production manager at ACME wants to understand the effects of different buffer sizes, cycle times and stoppage intervals. Through experimentation, he will then be able to focus on certain areas, to improve the process.

As the production manager is not a WITNESS expert, it will be easier to have this information read in from an external file. The following information needs to be read into your model.

- The drilling time per hole for OP10

- The Cycle Times for OP20

- The Capacities of the Buffers B1 - B6

- The Breakdown intervals and repair times for OP10 and OP20

- The Setup Intervals and times for OP10

### Exercise 6

#### Reading Data from Files - Part 2

The production manger at ACME wants to experiment with the order in which the operator at OP10 processes the parts to improve the process. A sample production schedule is given below. The manager needs to experiment with this information, therefore you need to store it in an external file. (Remember that the production manger does not know how to use WITNESS).

| Table 6-8: Exercise 6 Data |  |
| --- | --- |
| Part Type | Batch Size |
| A | 2 |
| B | 3 |
| A | 2 |
| C | 4 |
| B | 1 |

Having been completed, the schedule will be repeated.

| TIP: | During the first step, read the first line of the file and make sure that the machine is processing the correct part. Once you have completed these actions, begin reading the next line of the file and checking the next part. Continue in this way through the file. |
| --- | --- |

### Exercise 7

#### Creating a User Dialogue

Management would like to be able to interrupt a model run to obtain a report on machine utilization for either OP10 or OP20. The report should first ask which machine the manager would like a report for (i.e. OP10 or OP20) and then display the appropriate machine utilization as follows.

| OP10 | % Waiting Parts % Busy % Blocked | XX.XX XX.XX XX.XX |
| --- | --- | --- |

OR

| OP20(1) | % Waiting Parts % Busy % Blocked | XX.XX XX.XX XX.XX |
| --- | --- | --- |
| OP10(2) | % Waiting Parts % Busy % Blocked | XX.XX XX.XX XX.XX |

The manager should be able to remove the report from the screen by pressing ENTER on the keyboard.

### Exercise 8

#### Data Entry

- Read the following information in from an external file.

Cycle times:

OP10	mode = 2.75

OP20		1	3.75

2	5.10

BoxMaker

Packer

Capacity of Buffers

B1-B6

Quantity:

OP10

OP20

Fred

Joe

- After the data has been read in, offer the user the opportunity to amend the data using an Interact Box Front-End Menu.

## Case Study

### Hierarchical Model Display

An electronics assembly manufacturer wishes to build a simulation model of an essential part of their operation. You are part of a project team responsible for building the model that will be presented to other engineers. You will also present the model to individuals in upper management who will make the final decision to proceed with implementation of the solution proposed by the model.

Your project team has decided that the model display must be built using a hierarchical approach. This means that varying levels of detail in the model must be able to be displayed or hidden very quickly during the presentation depending on the audience’s preferences.

Build a model with the following specifications:

- Set up (rename) layers for separate display of three levels of detail of model Elements and related text and graphics: detail model view, work cell level view, and product line level view.

- Part Routing. Build a detailed model from the data in the table below. Display all graphics on the Simulation Layer:

| Table 6-9: Model Data |  |  |
| --- | --- | --- |
| Operation Description | Process Time (minutes) | Additional Display or Detail Information |
| WORK CELL #1 |  |  |
| SMT Component Dispensing: A batch of electronic components is processed 10 at a time. | 5.0 | Push the parts to a Pick & Place Machine when finished. |
| Pick & Place Machine: Parts are processed one at a time. | 4.0 | Push the parts to a Reflow Solder Machine when finished. |
| Reflow Wave Solder machine: Parts are individually wave soldered. | 3.0 | Push the parts to an Inline SMT Test Station when finished. |
| Inline SMT Test Station: Parts are individually tested. | 1.0 | Push the parts to the next work cell when finished. |
| WORK CELL #2 |  |  |
| SMT Assembly & Test Line: Parts from Work Cell #1 are processed individually. | 7.0 | Push to manual assembly workstation when finished. |
| Manual Assembly Workstation: Parts are individually processed. | 1.0 | Push to SMT Test Line when finished. |
| Final Assembly (automated): Parts are individually processed. | 5.0 | Push to Final (Functional) Test when finished. |
| Final (Functional) Test: Parts are individually tested then shipped. | 3.0 | Push to SHIP. |

- Arrange machines into the work cell groups noted above and shown in Figure 6-19.

- Draw outlines around the work cells, then around the entire product line on the appropriate layers as shown in Figure 6-19.

Figure 6-19: Model Layout Design

- Set up windows for separate display of the graphics for the Detail, Work Cell, and Product Line Levels.

- Add a button to the model display with descriptive text that by clicking on then on the  button will cause a new window displaying only the graphics for a particular level to open. Add buttons to display each of the three levels in separate windows. The buttons must be visible in any of these windows.

- In the Detail Level view, display the following statistics visible only in this window:

Timeseries graph displaying total system WIP.

Pie chart displaying utilization of the SMT Test Station.

- In the Work Cell Level view window, display the following statistics visible only in this window:

Percent time busy and blocked for all machines in Work Cell #1.

Percent time busy and blocked for all machines in Work Cell #2.

Timeseries graph with separate display of total WIP in Work Cell#1 and Work Cell #2.

- In the Product Line Level view window, display the following statistics visible only in this window:

Total units shipped.

Average unit throughput time in minutes.

- Create a separate window displaying graphics for all levels, but without any of the statistics for each of the levels described above. Create a separate button to activate this view window. It should be accessible from any view window.