WITNESS

Learning WITNESS

Lanner Group, Inc.
11000 Richmond Ave., Suite 680
Houston, Texas 77042

©1998 by Lanner Group, Inc.

All rights reserved. No part of this publication may be reproduced or transmitted in any form or by any means, including photocopying and recording, without written permission of Lanner Group, Inc. Application for permission should be addressed to the publisher. Such written permission must also be obtained before any part of this publication is stored in a retrieval system of any nature.

The information in this document is believed to be accurate. However, Lanner Group, Inc., assumes no responsibility for its use. The information is subject to change without notice and it must not be construed as a commitment by Lanner Group, Inc.

WITNESS is a trademark of Lanner Group, Inc.

IBM is a trademark of International Business Machines Corp.

MS-DOS, Windows, Microsoft Word, Microsoft Paint are trademarks of Microsoft Corp.

AutoCAD is a trademark of AutoDesk Inc.

Sentinel-Cplus is a trademark of Rainbow Technologies Inc.

This document published by Lanner Group, Inc., Houston, Texas.

Acknowledgments

The expertise and knowledge which went into making this manual, Learning WITNESS, required a special team of talented people. This material was developed through the conscientious and dedicated work from each person who spent long hours writing explanations and creating exercises which would promote a “real life” understanding of the simulation process.

Thanks to the trainers, technical support staff, programmers and consultants who added their parts to make this a successful project. I appreciate each person who worked extra to meet the impossibly short deadlines. Thank you everyone all for your work, skill, and knowledge.

A special thanks to the following:

Amana Refrigeration

Honeywell Micro Switch

Raytheon TI Systems, Terry Stockdale, P.E.

USA Lanner Group:

David Gaerte, Susan Hall, Christen Maltsburger, Pam Markt, Arvind Metha, Mike Meyer, Ray Masse, Rick Peterson

UK Lanner Group Team

Deborah De Mesa

Again, I thank everyone who participated in putting this project together.

Carlos H. Peredo

Project Manager

Senior Technical Consultant &
Regional Sales Manager for Latin America

Last Update: September 2, 1998

Table of Contents

Preface	i

Course Overview	i

Course Objectives	ii

Course Structure	ii

Terminology	iii

Introduction	1-1

Simulation Overview	1-2

What is Simulation?	1-2

Manual Simulation Exercise	1-3

The WITNESS Environment	1-6

Menus	1-7

Action Bars	1-8

Dialog Boxes and Property Sheets	1-10

Windows	1-12

Layers	1-12

The WITNESS Editor	1-13

Manipulating a Window	1-14

The Interact Box	1-16

The Clock (Time)	1-16

The Analog Clock	1-17

Copying, Cutting and Pasting	1-17

The Help Facility	1-18

Building WITNESS Models	2-1

Building a Model	2-2

The Three Steps to Model Building in WITNESS	2-2

The Basic Elements	2-3

Adding Elements to Your Model Using Designer Elements	2-4

Moving Parts (Entities) Between Elements	2-6

Display Forms	2-7

Detail Forms	2-11

Parts (Entities)	2-12

Buffers (Queues)	2-14

Machines (Activities)	2-17

Adding Elements Directly Into the Model	2-18

Running a Model in WITNESS	2-21

Getting Standard Reports from WITNESS	2-24

Modeling Stoppages	2-30

Simulating Stoppages	2-30

Physical Elements: Labor and Conveyors	2-35

Adding Labor Resource to Your Model	2-35

Conveyors	2-41

Adding Detail to Machines and Activities	2-45

Machines and Activity Types	2-45

Input and Output Rules	2-49

Examples	2-55

TV Manufacturer	2-55

A High-Volume Assembly Line	2-61

Exercises	2-67

Exercise 1	2-67

Exercise 2	2-68

Exercise 3	2-68

Exercise 4	2-69

Exercise 5	2-70

Exercise 6	2-70

Exercise 7	2-70

Enhancing Your Model	3-1

Sampling and Distributions	3-2

Principal of Sampling	3-2

Standard Theoretical Distributions (WITNESS Distributions)	3-3

Integer Distributions	3-4

Real Distributions	3-6

Using the Standard WITNESS Distributions	3-13

Using Your Own Data (User Distributions)	3-14

Adding Functionality to Your Model	3-17

Multi-Cycle Machines or Activities	3-17

Incorporating Shift Patterns	3-20

Action and Other Logical Elements	3-28

Actions	3-28

Attributes	3-35

Variables	3-47

Functions	3-60

New Designer Elements	3-66

Creating Designer Elements	3-66

Introducing the Designer Elements into a Model	3-69

Using Part (Entity) Routing to Describe the Flow Through the Model	3-69

Exercises	3-73

Exercise 1	3-73

Exercise 2	3-74

Exercise 3	3-74

Exercise 4	3-75

Exercise 5	3-75

Exercise 6	3-76

Exercise 7	3-76

Exercise 8	3-77

Exercise 9	3-77

Exercise 10	3-78

Exercise 11	3-79

Exercise 12	3-79

Case Study	3-81

Gadgets R Us Inc.	3-81

Simulation Projects	4-1

Methodology	4-2

Lanner Approach to Simulation Projects	4-2

Problem Definition	4-3

Model Building and Testing	4-4

Experimentation	4-4

Project Completion	4-8

Setting-Up a WITNESS Experiment	4-8

The Situations Page	4-9

The Run Control Page	4-11

The Replication Control Page	4-12

The Reporting Control Page	4-12

The Replication (All) Page	4-13

The Replication (Individual ) Page	4-15

Saving, Opening, and Running Experiment Files	4-16

Using Model & Status (SIM) Files	4-17

Using Model & Status (SIM) Files and WITNESS Command Language (WCL) Files.	4-19

Examples	4-20

The Importance of Selecting a Correct Warm-up Period	4-20

Experimenting with an Assembly Model	4-23

Graphical and Display Features	5-1

Graphical Features	5-2

Timeseries	5-2

Histograms	5-5

Pie Charts	5-8

Display Features	5-12

Icons and the Icon Editor	5-12

The Screen Editor	5-15

Specifying the Clock Display	5-17

Window Control and Window Association	5-19

Process Views	5-21

Element Flow and Meteor Trails	5-22

Display Layers and CAD Import	5-24

The WITNESS Virtual Screen	5-29

Presentation Enhancements	5-29

Using AVI Files	5-29

Controlling Windows Display During Model Execution	5-37

Use of Expression in Element Display and Screen Editor	5-38

Utilizing Layer Order of Display	5-39

Simulating the Movement of Parts by Labor	5-39

Improving Model Animation by Controlling Model Run Speed	5-40

Exercises	5-41

Exercise 1	5-41

Exercise 2	5-42

Exercise 3	5-42

Exercise 4	5-43

Case Study	5-43

Integrating a Facility Layout Drawing into Your Model	5-43

Advanced Features	6-1

Modules and Hierarchical Modeling	6-2

Introduction	6-2

Modules	6-2

Global Versus Local Element Names	6-3

The “Net” Method	6-5

The “Tree” Method	6-6

Creating a Module File and a Designer Element Module	6-7

Creating a Module from a Designer Element	6-10

Detail Options for a Simulation Module	6-13

Display tips for Hierarchical Models	6-17

Using Designer Elements Files (DES)	6-17

Last Comments on Hierarchical Modeling	6-19

Part or Link Files	6-19

Example of Using a Read Part File	6-21

Write Part File	6-22

Arrival Profiles	6-23

Files	6-26

Writing to a File	6-26

Reading from a File	6-28

Further Use of Actions	6-31

Commands and Constructs	6-31

Special Strings	6-32

User Actions	6-33

Immediate Actions	6-33

Example	6-34

Closed Loop Conveyor and Data Manipulation	6-34

Exercises	6-39

Exercise 1	6-39

Exercise 2	6-39

Exercise 3	6-40

Exercise 4	6-40

Exercise 5	6-41

Exercise 6	6-42

Exercise 7	6-43

Exercise 8	6-44

Case Study	6-45

Hierarchical Model Display	6-45

Progress Review	7-1

Example: Fan Blade Manufacturing	7-2

Introduction	7-2

Basic Process Steps	7-2

Detailed Description of Model	7-3

Problems Inherent in the Model	7-9

Possible Solutions	7-9

Other Items	7-11

Case Studies	7-20

ACME Manufacturing Co.	7-20

The ACME Boxmaker Model	7-27

Tracks and Vehicles	8-1

Introduction	8-2

Vehicles	8-2

Tracks	8-4

Loading and Unloading	8-6

Transfer Modes	8-8

Demand Driven Vehicle Systems	8-9

Introduction	8-9

Generating Demands	8-9

Using VSEARCH to Allocate Vehicles	8-10

Destination Rule	8-11

Using Work Search to Allocate Vehicles	8-12

The FINDV Statement	8-13

Zones and Capacities for tracks	8-14

Capacity	8-14

Zones	8-14

Parking Vehicles	8-15

Parking a Vehicle	8-15

Waking Up a Parked Vehicle	8-15

Action Commands and Built-in Functions	8-16

Indices for Tracks and Vehicles	8-16

Stop Time	8-16

Sequence of Events	8-17

Summaries	8-18

Advanced Tracks and Vehicles	8-19

Busy Time	8-19

Shifts for Vehicles	8-20

Exercises	8-22

Exercise 1	8-22

Exercise 2	8-23

Exercise 3	8-24

Exercise 4	8-24

Exercise 5	8-25

Exercise 6	8-25

Exercise 7	8-25

Exercise 8	8-26

Exercise 9	8-26

Exercise 10	8-27

Exercise 11	8-27

Exercise 12	8-29

Case Studies	8-30

International Airport Parking Lot Courtesy Bus Service	8-30

Automated Courtesy Bus System Feasibility Study	8-33

Continuous Simulation	9-1

Continuous Elements	9-2

Fluids	9-2

Pipes	9-4

Processors	9-6

Tanks	9-9

Controlling Fluid Flows	9-11

Basic Continuous Flow Rules	9-11

Flow Rule	9-11

Connect Rule	9-12

Least/Most Rules	9-12

If Rule	9-13

Move Fluid Command	9-13

The Recipe Rule	9-14

Element Details	9-15

Initial Contents and Fluid Changes Initial Contents	9-15

Labor and Shift Patterns	9-15

Stoppages	9-16

Breakdowns	9-16

Cleaning	9-18

Further Element Details	9-20

Actions	9-20

Built-In Functions	9-20

Warning Levels	9-21

Continuous Links to Discrete Elements	9-22

Continuous Flow Links with Discrete Elements	9-22

Modeling Issues	9-25

Monitor Interval	9-25

Mixture Interval	9-26

Flow Allocation	9-26

Examples	9-27

Representing Rolls of Material	9-27

Cola Manufacturing	9-33

Exercises	9-34

Exercise 1	9-34

Exercise 2	9-35

Exercise 3	9-36

Exercise 4	9-36

Exercise 5	9-37

Exercise 6	9-38

Case Studies	9-38

Poor’s Brewery	9-38

Brewery Line	9-41

Industry Studies	10-1

Assembling Electronic Components	10-1

An Example on how Simulation can be Used	10-1

Experimentation	10-5

Andover International Airport	10-7

Overview Of The Case	10-7

Creating The Model	10-7

Experimenting with Your Model	10-9

Dewey, Cheetam & Howe	10-11

Introduction	10-11

The Call Center	10-13

The Back Office	10-14

Experimentation	10-15

Auto Components Inc.	10-16

Summary	10-16

Ink Jet Cartridge Manufacture & Distribution	10-24

Introduction	10-24

Creating the Model	10-26

Experimenting with Your Model	10-29

Appendix A – Action Bars	11-1

Action Bar Buttons	11-2

Standard Action Bar	11-2

Advanced Action Bar	11-5

Execute Action Bar	11-7

The Prompting Action Bar	11-10

The Visual Rules Action Bar	11-10

The Selector Action Bar	11-10

Appendix B - Rules	12-1

Input and Output Rules	12-2

The Scope of Input and Output Rules	12-4

Labor Rules	12-5

Entering Rules by Using the Element Detail Dialogs	12-6

Input and Output Rules	12-6

Labor Rules	12-6

The Rule Editor	12-6

Labor Pre-emption	12-7

Visual Rules	12-7

Constraints	12-8

The Visual Input and Output Rules Toolbar	12-8

The Visual Labor Rules Toolbar	12-9

Rules Constraints	12-10

Comments in Rules	12-10

Rules and Indexed Elements	12-10

Use of Expressions within Rules	12-11

Use of Name Variables within Rules	12-11

Use of Functions Within Rules	12-12

Using IF with Rules	12-12

Consequences of Rule Failure	12-14

Parts	12-14

Vehicles	12-15

Fluids	12-15

Special Locations	12-16

Input Locations	12-16

Output Locations	12-16

Use of the NONE Keyword	12-17

Routing According to Part or Vehicle Type	12-17

Routing To and From Conveyors	12-18

Timing	12-19

Flow Allocation	12-20

Negative Flow Rates in Pipes	12-21

Rules Summary	12-21

WAIT	12-21

PUSH and PULL	12-22

Least and Most: Discrete Elements	12-23

LEAST and MOST: Continuous Processing Elements	12-25

Percent	12-26

Sequence	12-27

Select	12-28

Destination	12-29

Buffer	12-30

Flow	12-31

Connect	12-32

Recipe	12-33

Match (for machine Input Rules)	12-34

Match (for labor rules)	12-37

Appendix C - Actions	13-1

Introduction	13-1

When Actions Take Place	13-2

Stand Alone Actions	13-5

Building Actions	13-6

Using Actions Within Functions	13-7

Assignment - In Actions	13-8

= (equal sign character)	13-8

Part Control Actions	13-9

Change	13-9

Track and Vehicle Control Actions	13-10

CALL	13-11

DEST	13-11

DESTV	13-12

FINDV	13-12

VSEARCH	13-13

WAKEUP	13-13

Interact Box Input and Output Actions	13-14

PRINT	13-14

INPUT	13-16

OPENBOX	13-16

CLOSEBOX	13-17

INTERACT COLOR	13-17

INTERACT CURSOR	13-17

INTERACT CLEAR	13-18

File Input and Output Actions	13-19

READ	13-19

WRITE	13-20

REWIND	13-21

Modifying Simulation Elements	13-22

SET ICON	13-22

SET LIFE	13-23

SET QUANTITY	13-23

SET CAPACITY	13-25

Comments in Actions	13-25

! (exclamation mark)	13-25

Statistical Recording	13-27

RECORD	13-27

DRAWBAR	13-27

ADDBAR	13-28

Continuous Processing Elements Actions	13-29

CLEAN	13-29

PURGE	13-29

MOVE	13-30

Media Control Interface (MCI) Actions	13-31

MCIOPEN	13-31

MCICLOSE	13-32

MCIPLAY	13-33

MCIBEGIN	13-34

MCISTEP	13-34

MCI	13-34

Logical Expressions	13-35

Conditional Execution	13-37

IF…ELSE…ELSEIF…ENDIF	13-37

Branching	13-38

GOTO and LABEL	13-38

Repeating (Looping Constructs)	13-39

FOR…NEXT	13-39

WHILE…ENDWHILE…END	13-40

Terminating Actions	13-41

RETURN	13-41

STOP	13-41

Appendix D - WITNESS Built-In Functions	14-1

Real Functions	14-2

Integer Functions	14-10

Name Functions	14-14

String Functions	14-15

Void Functions	14-16

Element States	14-17

Display Types	14-21

Appendix E – Tips and Tricks	15-1

Random Selection	15-2

Using Tables for Cycle Times	15-3

Special Preventive Maintenance Case	15-4

Using GETPOSX and GETPOSY	15-9

Changing the Display Layer of an Existing Element or Background Graphic	15-14

Cloning a Model Element to a Designer Element	15-15

Maintaining Display Layer Integrity when Cloning an Element	15-16

Utilizing Multi-Dimensional Arrays	15-17

Operator Skill Level Affecting the Cycle Time of a Machine	15-21

Appendix F – FAQ’s	16-1

Appendix G – WITNESS Product History	17-1

Glossary	18-1

Index	18-1

Preface

## Course Overview

Simulation has much to offer any organization. The role of simulation is to evaluate alternatives that either support strategic initiatives, or support better performance at operational and tactical levels. Simulation provides the information needed to make these types of decisions. The simulation approach supports multiple analysis by allowing rapid changes to a model’s logic and data and is capable of handling large, complex systems; such as, a manufacturing facility.

WITNESS is Lanner Group’s simulation software package. It is the culmination of more than a decade of development. This experience has led us to evolve a visual, interactive and interpretative approach to simulation.

We at Lanner Group realize that simulation usage is growing and that new users in the marketplace fuel this growth. Therefore, this material was developed to provide a basic understanding about the benefits of using simulation in a business environment.

## Course Objectives

This course is designed to enable you to use WITNESS to quickly and confidently tackle simulation projects based on your own designs.

The slogan of this manual is simple: Easy to use, easy to understand, easy to read; therefore, easy to learn.

By the end of this course, you will be able to:

- List the basic WITNESS Elements and summarize their use

- Incorporate the WITNESS Elements into a model

- Move parts (entities) around the model

- Use control logic to change things in the model and collect statistics

- List the phases of simulation projects

- Run experiments and interpret the results

- Understand the basic characteristics and uses of Tracks and Vehicles

- Create Logic to load and unload Vehicles with parts (entities)

- Understand the various methods for allocating jobs to vehicles

- Summarize the characteristics and uses of continuous Elements

- Use fluid routing input and Output Rules

- Create simulation models for different industry sectors

## Course Structure

The purpose of this training course is to provide you with a solid understanding of simulation projects while using the WITNESS simulation software.

Chapter 1 contains a general overview of simulation including a discussion of the benefits of using an interactive simulation application. This chapter will also give you a clear understanding of how simulation works. In addition, it contains the information you need about the basic operation of WITNESS that you will use throughout this course.

Chapters 2, 3, 4, 5, and 6 contain the core course material, which concentrates on topics relating to Discrete-Event Simulations. These chapters provide you with the concepts necessary to create a basic model. You will learn how to create and use Elements effectively. In addition, Chapter 4 discusses simulation project management. Therefore, by the end of Chapter 6, you should be able to create and run models, generate reports, and interpret the results.

Chapter 7 is a review of Discrete-Event Simulation.

Chapter 8, Tracks, Vehicles, and Chapter 9, Continuous Simulation, are self-contained training modules. However, they both require the knowledge gained in chapters 1-7.

Chapter 10 is a group of case studies, which will help you to understand how simulation can be used in different industries.

## Terminology

This manual uses terminology that relates to both manufacturing and non-manufacturing environments since you can build models to simulate virtually any application. The following list may be useful in seeing the different terminology used in these two environments.

| Table 1-1: Manufacturing vs. General Terminology |  |  |
| --- | --- | --- |
| Manufacturing | General | Type |
| PART | ENTITY | Discrete |
| BUFFER | QUEUE | Discrete |
| MACHINE | ACTIVITY | Discrete |
| LABOR | RESOURCE | Both |
| FLUID | ITEM | Continuous |
| PROCESSOR | PROCESS | Continuous |
| TANK | STORE | Continuous |
| PIPE | STREAM | Continuous |
| PART FILE | LINK FILE | Discrete |
| BREAKDOWN | STOPPAGE | Both |
| REPAIR or DOWN | STOPPED | Both |
| ACTIONS ON REPAIR | ACTIONS ON RESUME | Both |
| CYCLE TIME | DURATION | Both |
| CYCLE LABOR | BUSY RESOURCE | Both |
| IDLE | FREE | Discrete |
| ASSEMBLY | JOIN | Discrete |
| PRODUCTION | SPLIT | Discrete |
| NUMBER SHIPPED | NUMBER SERVED | Both |
| NUMBER SCRAPPED | NUMBER LOST | Both |
| NUMBER REJECTED | DID NOT ENTER | Both |
| NUMBER ASSEMBLED | NUMBER JOINED | Both |
| WIP (Work In Progress) | NUMBER IN SYSTEM | Both |

Introduction

Objectives

- To provide an overview of simulation concepts.

- To learn the benefits and reasons for using simulation.

- To gain an understanding of how simulation works.

- To become familiar with the WITNESS user interface.

## Simulation Overview

### What is Simulation?

Developed in the 1950’s, simulation is the process of building a model that mimics reality. Discrete-Event Simulation, modeling a system over time, is the ability to model random events based on standard or non-standard distributions and to predict the complex interactions between these events.

A traditional definition of simulation is:

The act or process of simulating; feigning; the imitative representation of the functioning of one system or process by means of the functioning of another; examination of a problem often not subject to direct experimentation by means of a simulating device.

Simulation software designers generally define simulation as:

Imitating the operations of various kinds of real-world facilities or processes, the process of designing a mathematical-logical model of a real system and experimenting with this model on a computer.

#### What are the Benefits?

Using simulation to visualize the system under investigation increases the credibility of a project.

There are many other benefits to be gained through simulation modeling. These include:

- A greater understanding of the system being studied

- Improved communication of ideas

- Lower cost

- Ability to try many options quickly and easily

Simulation provides its users with an understanding of the system being modeled while avoiding the consequences of working with a live system. For example:

- The costs of building a proposed system

- Disturbing an existing system

- Destroying a system during stress testing

#### Why Simulate?

Simulation allows the user to monitor the dynamics of a system under various conditions. Simulation does not guarantee an optimal solution to any problem; it is the only appropriate analysis technique when formal mathematical methods cannot reflect the natural behavior of a system. Simulation provides:

- Risk reduction

- Greater understanding

- Operating cost reduction

- Capital cost reduction

- Ability to perform ‘what if?’ analyses

- Implementation of the best option

### Manual Simulation Exercise

Figure 1-1: Manual Simulation Exercise Layout

#### Description

This model, (See Figure 1-1) represents a four-machine cell which, on average, produces two types of components, A and B, in equal quantities. The arrival pattern of A's and B's is sequential: A arrives first, then a B, then an A and so forth. The first operation, machine M1, processes both types of parts and is fed from a large hopper. The machining time for M1 is 3 minutes. Components of type A are then processed by machine M2A, which has a cycle time of 7 minutes. Type B components are processed by machine M2B, which has a cycle time of 4 minutes. The final operation processes both component types and is carried out by machine M3, which has a cycle time of 4 minutes.

Discrete-Event Simulation steps through time, processing events in order. An event is defined as a change of state within the model.

The model used in this example is very simple, four machines and two parts. However, it becomes difficult to follow the model on paper as we move along the different time-frames. (See Table 1-2)

| Table 1-2: Simulation Example |  |  |
| --- | --- | --- |
| Time | Event | Cycle End Time |
| 0 | Component type A arrives Machine M1 starts operating No other change of state is possible Advance time to next event | 0 + 3 |
| 3 | Machine M1 finishes cycle Component A moves to machine M2A Machine M2A starts operating Machine M1 now available Component type B arrives Machine M1 starts operating No other change of state is possible Advance time to next event | 3 + 7 3 + 3 |
| 6 | Machine M1 finishes cycle Component B moves to machine M2B Machine M2B starts operating Component A arrives Machine M1 starts operating No other change of state is possible Advance time to next event | 6 + 4 6 + 3 |
| 9 | Machine M1 finishes cycle Machine M2A still operating, therefore component A unable to move to M2A Machine M1 Blocked until machine M2A finishes cycle No other change of state is possible Advance time to next event |  |
| 10 | Machine M2A finishes cycle Component A moves to machine M3 Machine M3 starts operating Machine M2B finishes cycle Component B unable to move to machine M3 Machine M2B Blocked until machine M3 finishes cycle Machine M2A now available Component A moves to machine M2A Machine M2A starts operating Machine M1 now unblocked and available Therefore component B arrives Machine M1 starts operating No other change of state is possible Advance time to next event | 10 + 4 10 + 7 10 + 3 |

Imagine how long it would take to follow this model of activity for a factory with 100 machines, multiple buffers, conveyors, and parts? This is the primary reason to use simulation programs like WITNESS; they keep track of the different events in the simulation. This allows you to spend time improving your process or processes rather than tracking the model itself.

There are two important observations we need to make regarding the previous table. Whenever numerous events take place at the same time, the computer program will give priority to the oldest event in the system, unless a specific priority is assigned in the model. In the event, where a machine (M1) has just finished processing a part, but it can not push it to next machine or buffer because it is busy or full, we say that the machine (M1) is blocked. Ideally, you want a system that runs without interruptions. Therefore, in some cases the goal of your simulation model will be to minimize the number of blockages in your system (See Table 1-2).

## The WITNESS Environment

Before going any further, we suggest you create your own working directory whenever you are using WITNESS. You should include in this directory a copy of the file START307.MOD or STARTUP.MOD, which you will find in the WITNESS DEMO directory. Once you open WITNESS, go to FileOpen and browse for either one of these files which are located in YOUR directory.

- The START307.MOD file is used for Discrete and Continuous modeling while the STARTUP.MOD is used for Discrete only.

Figure 1-2: File START307.MOD

The following overview briefly describes the various components of the WITNESS system.

### Menus

WITNESS uses Drop-Down Menus. Each line in the menu is a command. Seven different WITNESS menus are available: File, Edit, View, Model, Reports, Run, and Help.

Figure 1-3: File Drop-down Menu

General Menu Operation

- Place your pointer on any menu word. - Click the left mouse button. - Menu drops down.

- Move your pointer to any command. - The option becomes highlighted. - Click left mouse button. - The command is activated.

- When a dialog box appears on the screen, it takes priority over anything else displayed. For example, when using a form, you may not select another option from the menu.

### Action Bars

Action Bars help you quickly build WITNESS models. There are five Action Bars, each of which may be turned on or off independently. Three of the Action Bars, Standard, Advanced and Execute, contain icons which represent frequently used commands. These same commands are also available using the pull-down menus. The fourth Action Bar, Prompting, displays a description of the tools and menu options which you are about to use. The fifth Action Bar, Selector, enables you to list WITNESS commands, rules, and Elements and define your own Elements.

Figure 1-4: Activating or Deactivating Action Bars

Activate/Deactivate an Action Bar

- Select ViewAction Bars. - Select the Action Bar you need. The Action Bar appears.

Figure 1-5: The Standard Bar

Figure 1-6: The Advanced Bar

Figure 1-7: The Execute Bar

Figure 1-8: The Prompting Bar

Figure 1-9: The Selector Bar

- Each one of these bars is detailed in the Appendix.

### Dialog Boxes and Property Sheets

WITNESS uses of Microsoft® Windows™ dialog boxes. These dialog boxes are forms requesting or displaying information. They may contain a series of overlapping tabs. Each tab, when clicked, produces a property sheet.

Figure 1-10: A WITNESS Dialog Box

The following table lists the type of entries or fields that may be found in a dialog box.

| Table 1-3: Entry/Field Types |  |
| --- | --- |
| Type | Description |
| Input Fields | Places on a dialog box where you may enter or edit information. For example, in the Name field of a dialog box, you can change the name of the Element. - When the Element name is changed here, the name will be changed everywhere it is referenced. |
| Display Fields | Field which display information, which cannot be edited. |
| List-boxes | These special Display Fields contain multiple items. Open a list-box by: - Click on the down arrow to the right of the box. - The list appears. List-boxes have scroll bars down the right side of the box if the list is too long to be displayed in one window. Select any item from the list. |
| Buttons | Buttons on dialog boxes are usually single command activators. A typical dialog box contains the buttons, OK, Cancel, and Help. To select a button and execute the corresponding command: - Click on the button OR Press ENTER if the button is already highlighted. |
| OK | This button is used to tell WITNESS that you are satisfied with the contents of the dialog box. It closes the dialog box and updates the model according to the changes (if any) made. |
| Cancel | This button is used to tell WITNESS to close the dialog box and ignore any changes you have made. Pressing Esc on the keyboard usually has the same effect as selecting Cancel. |
| Notes | The Notes’ box allows you to document your model. |

### Windows

WITNESS provides an animated display of your simulation model that changes as the simulation run progresses. For example, machines or activities change color to indicate whether they are busy, broken down etc. Parts or entities can be seen moving from station to station as they are processed.

WITNESS models can become large and complex. Therefore, WITNESS provides a very large canvas to display this animation, and a number of windows through which this canvas may be viewed. We refer to this large canvas as the virtual screen. We refer to the image visible on your computer as the physical screen. The size of the virtual screen varies with different computers and display adapters.

WITNESS allows you to have several windows open on the screen at once. You may zoom in on the virtual screen, so that a small object fills your view, or zoom out to reveal a large portion of the virtual screen in less detail than normal. You may use multiple windows in order to draw attention to areas of the model, which appear in different parts of the virtual screen.

Windows should not be confused with dialog boxes. Windows are used to view objects on the virtual screen, and once opened, they remain open until you issue a command to close them. Dialog boxes are used information needs to pass between the user and WITNESS.

### Layers

WITNESS has a number of layers on which the Elements, text, pictures, lines, etc. can be drawn. The main layer is the Simulation Layer. By default, all items are placed on this layer. However, different levels of detail can be displayed on different layers and then switched on and off independently. This enables you to customize your display appropriate to the audience to which the model is being demonstrated. It is also possible to fix the display so that the display items cannot be moved around the screen.

### The WITNESS Editor

You can use the WITNESS editor to edit text in rules, actions, and the clipboard.

Figure 1-11: Sample WITNESS Editor

The WITNESS Editor operates like an input field. Click inside the edit area and begin typing. The following table describes some of the unique fields as well as the keyboard operations, which are active while you are in the Editor.

| Table 1-4: Editor Features |  |
| --- | --- |
| Type | Description |
| Prompt | When the prompt button is used, a dialog box is displayed which assists in the entry of rules and action commands. |
| Validate | Allows you to interactively validate the information you have entered in the rules editor. |
| Message Box | This box is used to display messages that indicate errors in rules or text information. If WITNESS finds a mistake in a rule or text you have entered, an appropriate message is displayed here. |
| Scrolling | Used to scroll through the text in the editor |
| Arrow keys | Used to move around the text on the screen |
| F1 | Help |
| Ctrl Home | Go to the top of the editor |
| Ctrl End | Go to the bottom of the editor |
| Home | Go to the start of a line |
| End | Go to the end of a line |
| Enter | Inserts a new line at the text cursor position and moves the text cursor to the start of the new line |
| Ctrl C | Copy the selected text |
| Ctrl X | Cut the selected text |
| Ctrl V | Paste selected text |
| Ctrl Z | Undo the last paste |
| Page up | Pages up to the previous page |
| Page down | Pages down to the next page |

### Manipulating a Window

An open window may be manipulated in a number of ways. The position of a window may be altered to reveal other objects on the physical screen. The size of a window may be changed in order to accommodate more than one window on the physical screen. A part of the model may be enlarged in order to draw attention to it by using the zoom-in feature, or a larger part of the virtual screen may be revealed using the zoom out feature.

| Table 1-5: Window Operations |  |
| --- | --- |
| Type | Description |
| To move a window | Hold the mouse button down in the header bar and drag the window. |
| To resize a window | Drag the window borders. |
| To scroll the virtual screen | Click in a slider bar above or below the slider itself. This moves across the virtual screen by a distance equivalent to the size of the window. |
| To locate a position within the virtual screen | Drag the slider until the window displays the virtual screen at the required position. |
| To make a window active | Click inside the border of the window that you wish to make active. |
| To zoom in | Make the window active and press the + key. |
| To zoom out | Make the window active and press the – key. |
| To close a window | Click on the box in the top right of the window. Alternatively, close the window by using the ViewWindow menu option and select the window to close. |
| To re-open a window | Select the appropriate window from the ViewWindow menu option. A window may be maximized (made to fill the screen) by clicking on the maximize icon in the top right of the window. To restore its previous position, click again on this icon. |

The options of a window such as the name and the color of the background can be modified.

Windows Options

- Select the window to modify.

- Press on “ * ” from your keyboard OR ViewWindowsControl. - The Window Control Window pops up.

- Make your changes and click OK.

Figure 1-12: Window Control

### The Interact Box

The Interact Box is used to exchange information with the user concerning the running of a model (ViewWindowInteract Box).

Figure 1-13: Interact Box

### The Clock (Time)

The Time window is a digital display of the current simulation time (ViewWindowTime).

Figure 1-14: Digital Time

### The Analog Clock

The Analog Clock window displays the current simulation time in analog format (ViewWindowClock).

Figure 1-15: Analog Clock

### Copying, Cutting and Pasting

You can use the copy, cut, and paste facility as a shortcut for entering similar information in more than one location. Copied and cut text is displayed in the clipboard and may be edited. Text can be also imported from a file to the clipboard, ready for pasting, and exported from WITNESS to the WINDOWS clipboard for pasting to a file.

Copy and Cut Element’s Display or Detail Features

- Select ModelElements

- Use the standard Windows copying and pasting methods.

- Select EditClipboard Text. - You can now edit the clipboard text.

Types of information that you can copy and paste:

- Text in notes

- Text in input and Output Rules

- Text in actions

- Text in labor rules

- Graphics drawn on the screen editor

- Element display information

- Element detail information

### The Help Facility

The Help menus are similar to other on-line help systems. The usual menus and search facilities are available.

Figure 1-16: The Help System

| Table 1-6: Help Commands |  |
| --- | --- |
| Command | Description |
| Contents | Summons the WITNESS Help contents screen. - Pressing the F1 key has the same effect as selecting contents unless you are in a WITNESS dialog box. When you press F1 from within a dialog box, the help you get is context-sensitive to the dialog box. |
| Search for help on... | Takes you to a search field. If you type a topic or just the first two or three letters of a topic, the Help system lists the related topic items if they exist. |
| Glossary | Glossary of WITNESS terms. |
| Technical Support | Provides useful information about where to get support. |
| About WITNESS | Displays the WITNESS copyright message and version number. Click on the OK button or press ENTER to remove this message from the screen. |

Webster’s Ninth New Collegiate Dictionary, Merriam-Webster, Inc., 1983

Operations Research: An Introduction, H.A.Taha, Macmillan, 1971