# Lab 7.5.2: Challenge Wireless Configuration

## Topology Diagram

## Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Fa0/1.1 | 172.17.1.1 | 255.255.255.0 | N/A |
|  | Fa0/1.10 | 172.17.10.1 | 255.255.255.0 | N/A |
|  | Fa0/1.20 | 172.17.20.1 | 255.255.255.0 | N/A |
|  | Fa0/1.88 | 172.17.88.1 | 255.255.255.0 | N/A |
|  | Lo0 | 10.1.1.1 | 255.255.255.252 | N/A |
| WRS2 | WAN | 172.17.88.25 | 255.255.255.0 | 172.17.88.1 |
|  | LAN/Wireless | 172.17.40.1 | 255.255.255.0 | N/A |
| WRS3 | WAN | 172.17.88.35 | 255.255.255.0 | 172.17.88.1 |
|  | LAN/Wireless | 172.17.30.1 | 255.255.255.0 | N/A |
| PC1 | NIC | 172.17.10.21 | 255.255.255.0 | 172.17.10.1 |
| PC2 | NIC | 172.17.20.22 | 255.255.255.0 | 172.17.20.1 |

## Learning Objectives

Upon completion of this lab, you will be able to:

- Configure switch port VLAN information and port security.

- Hard reset a Linksys WRT300N router.

- Connect and verify connectivity to a wireless router.

- Navigate to a Linksys WRT300N’s web utility page.

- Configure the IP settings of a Linksys WRT300N.

- Configure DHCP on a Linksys WRT300N.

- Configure static routes on both standard Cisco routers and on a WRT300N.

- Change the network mode and corresponding network channel on a WRT300N.

- Enable WEP encryption and disable SSID broadcast.

- Enable a wireless MAC filter.

- Configure access restrictions on a WRT300N.

- Configure router management password on a WRT300N.

- Enable logging on a WRT300N.

- Upgrade WRT300N firmware.

- Learn diagnosis, backup, restore, and confirmation mechanisms on a WRT300N.

## Scenario

In this lab, you will configure a Linksys WRT300N, port security on a Cisco switch, and static routes on multiple devices. Make note of the procedures involved in connecting clients to a wireless network. Some configuration changes will cause clients to disconnect. These clients then have to reconnect after making changes to the configuration.

Task 1: Perform Basic Router Configurations

Step 1: Physically connect the devices based on the topology diagram.

Step 2: Configure R1 according to the following guidelines:

- Configure router hostname.

- Disable DNS lookup.

- Configure privileged EXEC password of Cisco.

- Configure FastEthernet 0/1 and its subinterfaces.

- Configure Loopback0.

- Configure synchronous logging, exec-timeout, and a password of cisco on the console port.

Task 2: Configure Switch Interfaces

Configure switch hostnames on S1, S2, and S3. Set the switches to transparent, clear the VLAN information, and create VLANs 10, 20, and 88.

Step 1: Configure switch port interfaces on S1, S2, and S3.

Configure the interfaces on the S1, S2, and S3 switches with the connections from topology diagram.

Configure connections between two switches configure trunks.

Configure connections to a wireless router as access mode for VLAN 88.

Configure S2’s connection to PC1 in VLAN 10 and PC2’s connection in VLAN 20.

Configure S1’s connection to R1 as a trunk.

Allow all VLANS across trunking interfaces.

Step 2: Verify VLANs and trunking.

Use the show ip interface trunk command on S1 and the show vlan command on S2 to verify that the switches are trunking correctly and the proper VLANs exist.

S1#show interface trunk

Port        Mode         Encapsulation  Status        Native vlan

Fa0/1       on           802.1q         trunking      1

Fa0/2       on           802.1q         trunking      1

Fa0/3       on           802.1q         trunking      1

Fa0/4       on           802.1q         trunking      1

Fa0/5       on           802.1q         trunking      1

Port      Vlans allowed on trunk

Fa0/1       1-4094

Fa0/2       1-4094

Fa0/3       1-4094

Fa0/4       1-4094

Fa0/5       1-4094

Port        Vlans allowed and active in management domain

Fa0/1       1,10,20,88

Fa0/2       1,10,20,88

Fa0/3       1,10,20,88

Fa0/4       1,10,20,88

Fa0/5       1,10,20,88

Port        Vlans in spanning tree forwarding state and not pruned

Port        Vlans in spanning tree forwarding state and not pruned

Fa0/1       1,10,20,88

Fa0/2       none         -- blocked due to STP – varies based on root

Fa0/3       1,10,20,88

Fa0/4       1,10,20,88

Fa0/5       1,10,20,88>

S2#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -----------------------------

1    default                          active    Fa0/5, Fa0/6, Fa0/8, Fa0/9

Fa0/10, Fa0/12, Fa0/13,Fa0/14

Fa0/15, Fa0/16, Fa0/17,Fa0/19

Fa0/20, Fa0/21, Fa0/22,Fa0/23

Fa0/24, Gi0/1, Gi0/2

10   VLAN0010                         active    Fa0/11

20   VLAN0020                         active    Fa0/18

88   VLAN0088                         active    Fa0/7

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

When you have finished, be sure to save the running configuration to the NVRAM of the router and switches.

Step 3: Configure the Ethernet interfaces of PC1 and PC2.

Configure the Ethernet interfaces of PC1 and PC2 with the IP addresses and default gateways according to the addressing table at the beginning of the lab.

Step 4: Test the PC configuration.

Ping the default gateway from the PC: 172.17.10.1 for PC1, and 172.17.20.1 from PC2.

Go to Start->Run->cmd and type ping 172.17.x.x

Task 3: Connect to the Linksys WRT300N Router WRS3

Check with your instructor that the wireless router has its factory default settings. If it does not, you must hard reset the router. To do so, find the reset button on the back of the router. Using a pen or other thin instrument, hold down the reset button for 5 seconds. The router should now be restored to its factory default settings.

Step 1: Connect to the wireless router.

When the wireless router is returned to its default configuration, it will broadcast the default SSID of “linksys”. Step 1: Use Windows XP to connect to the wireless router.

Note: Before attempting to connect to the WRS3 router, make sure that the WRS2 router’s power cord is unplugged. Having both wireless routers powered on will cause the PC to find two wireless networks with an SSID of “linksys”, making it difficult to distinguish which router you are trying to connect to.

Locate the Wireless Network Connection icon in your taskbar, or go to Start > Control Panel > Network Connections. Right-click the icon and select View Available Wireless Networks.

You are prompted with the following display. Note that the factory default SSID of the router is simply “Linksys.”

Select Linksys and click Connect.

After a period of time you will be connected.

Step 2: Verify connectivity settings.

Verify the connectivity settings by going to Start > Run and typing cmd. At the command prompt, type the command ipconfig to view your network device information. Notice which IP address is the default gateway. This is the default IP address of a Linksys WRT300N.

Task 4: Configure the WRS3 Using the Web Utility

Step 1: Go to the default URL.

Using a web browser, navigate to http://192.168.1.1 which is the default URL for the WRT300N.

Step 2: Enter authentication information.

You are prompted for a username and password. Enter the WRT300N factory default password of admin and leave the username field blank.

You should now be viewing the default page of the Linksys WRT300N web utility.

Task 5: Configure IP Settings for the Linksys WRT300N

The best way to understand the difference between the Internet Setup and the Network Setup options is to think of the WRT300N as being similar to a Cisco IOS-based router with two separate interfaces. One of the interfaces, the one configured under Internet Setup, acts as the connection to the switches and the rest of the network. This connection would eventually lead out to the Internet, although in our topology there is no connection to the Internet. The other interface, configured under Network Setup, acts as the interface connecting to clients, both wireless and wired.

Step 1: Set the Internet connection type to static IP.

Step 2: Set the IP address settings for Internet Setup.

- Set the Internet IP Address to 172.17.88.35.

- Set the Subnet Mask to 255.255.255.0.

- Set the Default Gateway to 172.17.88.1 (the FastEthernet 0/1 VLAN 88 IP address of R1).

Step 3: Configure the Network Setup IP address to 172.17.30.1.

Step 4: Save the settings.

Click Save Settings. You are prompted to click Continue. Since you are connected wirelessly, you will not be redirected to the new URL of the web utility (http://172.17.30.1).

In order for the new IP address changes to take place, the PC has to release its old IP address and dynamically acquire a new one from the 172.17.30.0/24 network.

Step 5: Release the old Network Setup IP Address

In command prompt, use the command ipconfig /release to release the current DHCP address. To get a new IP address in the new network, issue the command ipconfig /renew. A new IP address should be pulled from the 172.17.30.0/24 network.

Step 6: View the PC IP configuration settings.

Go to command prompt and use the command ipconfig. If the address has not been updated to the 172.17.30.0/24 network, it will be necessary to release and renew the IP address on the client.

Step 7: Go to the new URL and enter authentication information.

In your favorite web browser, navigate to http://172.17.30.1 which is the new URL for the WRT300N. Enter the default username and password when you are prompted to do so.

Task 6: Configure DHCP Settings and Router Time Zone Settings

Step 1:	Give PC6 a static DHCP binding.

From the Basic Setup page in the Network Setup section, click DHCP Reservations. Find PC6 in the list of current DHCP clients. (Note that your PC may have a different name.) Click the check box in the correct row for the PC and then click Add Clients.

This gives PC6, the computer with a MAC address of 00:05:4E:49:64:F8, the same IP address, 172.17.30.100, whenever it requests an address through DHCP. This is only an example of a quick way to permanently bind a client to its current DHCP-given IP address. Now, you will assign PC6 the IP address in the topology diagram, not the one it received initially. Click Remove to assign a new address.

Step 2: Assign PC6 the 172.17.30.26 address.

By entering the PC6 address under Manually Adding Client, whenever PC6 connects to the wireless router, it receives the IP address 172.17.30.26 via DHCP. Save your changes.

Step 3: Verify the static IP address change.

Since we already have an IP address from DHCP we are not going to get the new address, 172.17.30.26, until we reconnect. We will wait and notice that later in Task 7, Step 6 and verify that this change has taken place.

Step 4: Configure the DHCP server.

Set the start address to 50, the maximum number of users to 25, and the lease time to 2 hours (or 120 minutes).

These settings give any PC that connects to this router wirelessly requesting an IP address through DHCP, an address between 172.17.30.50–74. Only 25 clients at a time are able to get an IP address and can only have the IP address for two hours, after which time they must request a new one.

Note: IP Address Range does not update until you click Save Settings.

Step 5: Configure the router for the appropriate time zone.

At the bottom of the Basic Setup page, change the time zone of the router to reflect your location.

Step 6: Save your settings!

Click Save Settings. You are prompted to click Continue.

Task 7: Basic Wireless Settings

Step 1: Navigate to the Wireless page and set the network mode in the Basic Wireless Settings tab.

The Linksys WRT300N allows you to choose in which network mode to operate. Currently, the most used network mode for clients is Wireless-G and for routers is BG-Mixed. When a router is operating in BG-Mixed, it can accept both B and G clients. However, if a B client connects, the router must scale down to the slower level of B. For this lab, we are assuming all clients are running B only, so choose Wireless-B Only.

Step 2: Configure other settings.

Change the Network Name (SSID) to WRS3_[number], where the number is a unique ID number given to you by your instructor. Change the Standard Channel to the channel assigned to you by your instructor, and disable SSID Broadcast.

Why is it good to change the wireless channel to be different from the default channel?

Why is it recommended to disable SSID broadcast?

Step 3: Save Settings.

Select the Save Settings link to save all changes. Click Continue to continue to the next task.

Step 4: Verify that the SSID of the router is no longer being broadcast.

Scan for wireless networks. Locate the Wireless Network Connection icon in your taskbar, or go to Start > Control Panel > Network Connections. Right-click the icon and select View Available Wireless Networks.

Does the SSID of the wireless router appear?

Step 5: Reconnect to the wireless network.

Navigate to Start > Control Panel > Network Connections, right-click the Wireless Network Connection icon, and select Properties.

In the Wireless Networks tab, select Add.

In the Association tab, enter WRS3_[number] as the SSID, and set the Data Encryption to Disabled. Select OK, and then select OK again. Windows should now try to reconnect to the wireless router.

Step 6: Verify the settings.

Now that you have reconnected to the network, you have the new DHCP settings that you configured in Task 6, Step 2. Verify this at the command prompt of PC6 with the ipconfig command.

Task 8: Enable Wireless Security

Step 1: Reconnect to the router setup page (http://172.17.30.1).

Step 2: Navigate to the Wireless page and then select the Wireless Security tab.

Step 3: Under Security Mode, select WEP.

Step 4: Enter a WEP key.

A network is only as secure as its weakest point, and a wireless router is a very convenient place to start if someone wants to damage your network. By not broadcasting the SSID and requiring a WEP key to connect to the router, you are adding a few levels of security.

Unfortunately, there are tools that can discover networks that are not even broadcasting their SSID, and there are even tools that can crack WEP key encryption.

Add the WEP key 1234567890 as Key 1.

Step 5: Save your settings.

Now that WRS3 has been configured with WEP security, and PC6 is not configured with WEP, you will be disconnected from the network.

Step 6: Configure Windows to use WEP authentication.

Navigate to the Network Connections page again and right-click the Wireless Network Connection icon. In the Wireless Networks tab, locate the WRS3 network, and click Properties.

- Set Data Encryption to WEP.

- Uncheck This Key Is Provided For Me.

- Enter the network key of 1234567890, as configured before on the router.

- Click OK and OK.

Windows should now reconnect to the network.

Task 9: Configure a Wireless MAC Filter

Step 1: Add a MAC filter.

- Navigate back to the web utility page of the router (http://172.17.30.1).

- Navigate to the Wireless page and then to the Wireless MAC Filter tab.

- Check Enabled.

- Select Prevent PCs listed below from accessing the wireless network.

- Enter the MAC address 00:05:4E:49:64:87.

- Click Save Settings.

This prevents any client with the MAC address 00:05:4E:49:64:87 from accessing the wireless network.

Step 2: Click Wireless Client List.

The Wireless Client List shows anyone currently connected to the router via a wireless connection. Also take note of the option Save to MAC filter list. Checking this option automatically adds the MAC address of that client to the list of MAC addresses to prevent or permit access to the wireless network.

What is an extremely robust way of only allowing clients of your choosing to connect to the wireless network?

Why is this not feasible in large networks?

What is a convenient way of adding MAC addresses if everyone to whom you wanted to allow access was already connected to the wireless network?

Task 10: Setting Access Restrictions

Configure an access restriction that prevents Telnet access Monday through Friday to users getting a DHCP address from the preset pool (172.17.30.50 – 74).

Step 1: Navigate to the Access Restrictions tab.

In the Access Restrictions tab, set the following:

- Policy Name – No_Telnet

- Status – Enabled

- Access Restriction – Allow

- Schedule – Uncheck Everyday and recheck Monday through Friday

- Blocked Applications – Add Telnet to Blocked List

Step 2: Set the IP address range.

Apply this configuration to anyone that is using a default DHCP address in the range of 172.17.30.50 – 74.

Click the Edit List button at the top of the window and enter the IP address range. Save the settings.

Click the Save Settings button to save the access restriction settings. Click Close to close the window and continue with the next task.

Task 11: Managing and Securing the Web Utility of the Router

Step 1: Configure web access.

Navigate to the Administration section. Change the router password to cisco.

For Web Utility Access, select both HTTP and HTTPS. Selecting HTTPS access allows a network administrator to manage the router via https://172.17.30.1 with SSL, a more secure form of HTTP. If you choose to do this in the lab, you may have to accept certificates.

For Web Utility Access via Wireless, select Enabled. If you disabled this option, the Web Utility would not be available to clients connected wirelessly. Disabling access is another form of security, because it requires the user to be directly connected to the router before changing settings. However, in this lab scenario, you are configuring the router via wireless access, so disabling access would not be a good idea!

Click the Save Settings option at the bottom of the page. You may be prompted for the configured password. Enter cisco for the password and reconnect.

Now back up your configuration by clicking the Backup Configurations button. When prompted, save the file to your desktop.

Step 2: Restore your configuration.

If your settings are accidentally or intentionally changed or erased, you can restore them from a working configuration using the Restore Configurations option located in the Backup and Restore section.

Click the Restore Configuration button now.  In the Restore Configurations window, browse to the previously saved configuration file. Click the Start to Restore button. Your previous settings should be successfully restored.

Step 3: Enable logging.

Navigate to the Log tab of the Administration section and enable logging. You are now able to view the log of the router.

Step 4: Save your settings.

Task 12: Creating and Verifying Full Connectivity

Step 1: Filter anonymous Internet requests.

In the Security page, uncheck Filter Anonymous Internet Requests. Disabling this option allows you to ping the WRS3 internal LAN/wireless IP address, 172.17.30.1, from places connected to its WAN port. Don’t forget to Save your settings.

Step 2: Disable NAT.

In the Setup page, click the Advanced Routing tab. Disable NAT. Don’t forget to Save your settings.

Step 3: Connect to WRS2.

Now that WRS3 has been configured, it no longer broadcasts the default SSID of linksys. Power up the WRS2 wireless router and perform similar configurations. Review previous steps to connect PC3 to WRS2 via a wireless connection.

Set the IP address settings for Internet Setup.

- Set the Internet IP address to 172.17.88.25.

- Set the Subnet Mask to 255.255.255.0.

Set the Default Gateway to the FastEthernet 0/1 VLAN 88 IP address of R1, 172.17.88.1.

Configure the Network Setup IP address to 172.17.40.1.

Statically bind the MAC address of PC3 to the DHCP address 172.17.40.23.

Change the wireless SSID to WRS2_[number].

Step 4: Configure R1 with static routes to the 172.17.30.0 and 172.17.40.0 networks.

R1(config)#ip route 172.17.30.0 255.255.255.0 172.17.88.35

R1(config)#ip route 172.17.40.0 255.255.255.0 172.17.88.25

Step 5: Repeat steps 1 and 2 above for WRS2.

Disable the Filter anonymous Internet requests.

Disable NAT.

Step 6: Verify connectivity.

Verify that R1 has routes to PC3 and PC6 and that it can successfully ping them.

R1#sh ip route

<output deleted>

Gateway of last resort is not set

172.17.0.0/24 is subnetted, 5 subnets

S       172.17.40.0 [1/0] via 172.17.88.25

S       172.17.30.0 [1/0] via 172.17.88.35

C       172.17.20.0 is directly connected, FastEthernet0/1.20

C       172.17.10.0 is directly connected, FastEthernet0/1.10

C       172.17.88.0 is directly connected, FastEthernet0/1.88

10.0.0.0/30 is subnetted, 1 subnets

C       10.1.1.0 is directly connected, Loopback0

R1#ping 172.17.30.26

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 172.17.30.26, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/4 ms

R1#ping 172.17.40.23

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 172.17.40.23, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/4 ms

Verify that PC3 and PC6 can ping the loopback of R1.

Verify that PC3 and PC6 can ping each other.

Verify that PC3 and PC6 can ping PC1 and PC2.

Task 13: Configuring Routing Efficiency

Step 1: Use Traceroute to view the network connection.

Because R1 is the default gateway, the Linksys router goes to R1 to get to a network it does not know how to get to, including the clients of the other Linksys routers.

A packet from PC3 to PC6 first reaches its default gateway of 172.17.40.1, then it is sent out the WRS2 WAN interface of 172.17.88.25 toward the WRS2 default gateway (172.17.88.1). From there, R1 send the packet to the WRS3 WAN interface, 172.17.88.35, where WRS3 handles it.

On WRS2, you can verify this in the Diagnostics tab in the Administration section. In the Traceroute Test field, enter the IP address of PC6, 172.17.30.26

Now click Start to Traceroute, a pop-up will appear.

If WRS2 knew that it could get to the 172.17.30.0 network from 172.17.88.35 it would just directly send it to that IP address.  So let’s tell it!

Step 2: Configure a new route.

On WRS2, on the Setup page, click the Advanced Routing tab. For Static Routing, enter the following settings:

- In the Route Name field, enter To WRS3 Clients.

- For Destination LAN IP, enter the network behind WRS3: 172.17.30.0.

- Enter a subnet mask of 255.255.255.0.

- Enter a gateway of 172.17.88.35.

- Set the interface to Internet (WAN).

- Save your settings.

Step 3: Verify the new route.

In the Diagnostics tab in the Administration section, re-enter the IP address of PC3 in the Traceroute Test field. Click Start to Traceroute to see the route.

Notice WRS2 goes straight to WRS3 and saves us the extra hop to R1!

Do the same thing on WRS3 for the 172.17.40.0/24 network, pointing towards WRS2’s WAN interface, 172.17.88.25.

Task 14: Configuring Port Security

Step 1: Configure PC1 port security.

Log on to switch S2. Configure the PC1 switch port, FastEthernet 0/11 with port security, and enable dynamic sticky MAC addresses.

Step 2: Configure PC2 port security.

Repeat for FastEthernet 0/18.

Step 3: Generate traffic across the ports by pinging PC2 from PC1.

Step 4: Verify port security.

S2#show port-security address

Secure Mac Address Table

------------------------------------------------------------------------

Vlan    Mac Address       Type                     Ports   Remaining Age

(mins)

----    -----------       ----                     -----   -------------

10    0006.5b1e.33fa    SecureSticky             Fa0/11       -

20    0001.4ac2.22ca    SecureSticky             Fa0/18       -

------------------------------------------------------------------------

Total Addresses in System (excluding one mac per port)     : 0

Max Addresses limit in System (excluding one mac per port) : 6272

S2#show port-security interface FastEthernet 0/11

Port Security              : Enabled

Port Status                : Secure-up

Violation Mode             : Shutdown

Aging Time                 : 0 mins

Aging Type                 : Absolute

SecureStatic Address Aging : Disabled

Maximum MAC Addresses      : 1

Total MAC Addresses        : 1

Configured MAC Addresses   : 0

Sticky MAC Addresses       : 1

Last Source Address:Vlan   : 0006.5b1e.33fa:10

Security Violation Count   : 0

Task 15: Restore WRT300N routers to factory defaults

Step 1: Clear settings of both WRT300N routers.

In order to clear both of the WRT300N routers to their factory defaults, navigate to the Administration page, click on Factory Defaults, and click the Restore All Settings button.