# Packet Tracer – CCNA Skills Integration Challenge

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| HQ | G0/0 | 10.0.1.1 | 255.255.255.0 |
|  | G0/1 | 192.0.2.1 | 255.255.255.0 |
|  | S0/0/0 | 10.255.255.1 | 255.255.255.252 |
|  | S0/0/1 | 10.255.255.253 | 255.255.255.252 |
|  | S0/1/0 | 209.165.201.1 | 255.255.255.252 |
| B1 | G0/0.10 | 10.1.10.1 | 255.255.255.0 |
|  | G0/0.20 | 10.1.20.1 | 255.255.255.0 |
|  | G0/0.30 | 10.1.30.1 | 255.255.255.0 |
|  | G0/0.99 | 10.1.99.1 | 255.255.255.0 |
|  | S0/0/0 | 10.255.255.2 | 255.255.255.252 |
| B1-S2 | VLAN 99 | 10.1.99.22 | 255.255.255.0 |

VLAN Configurations and Port Mappings

| VLAN Number | Network Address | VLAN Name | Port Mappings |
| --- | --- | --- | --- |
| 10 | 10.1.10.0/24 | Admin | F0/6 |
| 20 | 10.1.20.0/24 | Sales | F0/11 |
| 30 | 10.1.30.0/24 | Production | F0/16 |
| 99 | 10.1.99.0/24 | Mgmt&Native | F0/1-4 |
| 999 | N/A | BlackHole | Unused Ports |

Scenario

In this comprehensive CCNA skills activity, the XYZ Corporation uses a combination of eBGP and PPP for WAN connections. Other technologies include NAT, DHCP, static and default routing, EIGRP for IPv4, inter-VLAN routing, and VLAN configurations. Security configurations include SSH, port security, switch security, and ACLs.

Note: Only HQ, B1, B1-S2, and the PCs are accessible. The user EXEC password is cisco and the privileged EXEC password is class.

Requirements

PPP

Configure the WAN link from HQ to the Internet using PPP encapsulation and CHAP authentication.

Create a user ISP with the password of cisco.

Configure the WAN link from HQ to NewB using PPP encapsulation and PAP authentication.

Create a user NewB with the password of cisco.

Note: The ppp pap sent-username is not graded by Packet Tracer. However, it must be configured before the link will come up between HQ and NewB.

eBGP

Configure eBGP between HQ and the Internet.

HQ belongs to AS 65000.

The IP address for the BGP router in the Internet cloud is 209.165.201.2.

Advertise the 192.0.2.0/24 network to the Internet.

NAT

Configure dynamic NAT on HQ

Allow all addresses for the 10.0.0.0/8 address space to be translated using a standard access list named NAT.

XYZ Corporation owns the 209.165.200.240/29 address space. The pool, HQ, uses addresses .241 to .245 with a /29 mask. Bind the NAT ACL to the pool HQ. Configure PAT.

The connections to the Internet and HQ-DataCenter are outside XYZ Corporation.

Inter-VLAN Routing

Configure B1 for inter-VLAN routing.

Using the addressing table for branch routers, configure and activate the LAN interface for inter-VLAN routing. VLAN 99 is the native VLAN.

Static and Default Routing

Configure HQ with a static route to the NewB LAN. Use the exit interface as an argument.

Configure B1 with a default route to HQ. Use the next-hop IP address as an argument.

EIGRP Routing

Configure and optimize HQ and B1 with EIGRP routing.

Use autonomous system 100.

Disable EIGRP updates on appropriate interfaces.

VLANs and Trunking Configurations

Note: Logging to the console is turned off on B1-S2 so that the Native VLAN mismatch messages will not interrupt your configurations. If you would prefer to view console messages, enter the global configuration command logging console.

Configure trunking and VLANs on B1-S2.

Create and name the VLANs listed in the VLAN Configuration and Port Mappings table on B1-S2 only.

Configure the VLAN 99 interface and default gateway.

Set trunking mode to on for F0/1 - F0/4.

Assign VLANs to the appropriate access ports.

Disable all unused ports and assign the BlackHole VLAN.

Port Security

Use the following policy to establish port security on the B1-S2 access ports:

Allow two MAC addresses to be learned on the port.

Configure the learned MAC addresses to be added to the configuration.

Set the port to send a message if there is a security violation. Traffic is still allowed from the first two MAC addresses learned.

SSH

Configure HQ to use SSH for remote access.

Set the modulus to 2048. The domain name is CCNASkills.com.

The username is admin and the password is adminonly.

Only SSH should be allowed on VTY lines.

Modify the SSH defaults: version 2; 60-second timeout; two retries.

DHCP

On B1, configure a DHCP pool for the Sales VLAN 20 using the following requirements:

Exclude the first 10 IP addresses in the range.

The case-sensitive pool name is VLAN20.

Include the DNS server attached to the HQ LAN as part of the DHCP configuration.

Configure the Sales PC to use DHCP.

Access List Policy

Because HQ is connected to the Internet, configure and apply a named ACL called HQINBOUND in the following order:

Allow inbound BGP updates (TCP port 179) for any source to any destination.

Allow inbound HTTP requests from any source to the HQ-DataCenter network.

Allow only established TCP sessions from the Internet.

Allow only inbound ping replies from the Internet.

Explicitly block all other inbound access from the Internet.

Connectivity

Verify full connectivity from each PC to WWW.pka and www.cisco.pka.

The Outside Host should be able to access the webpage at WWW.pka.

All the test in Scenario 0 should be successful.