Packet Tracer – Skills Integration Challenge

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP-1 | S0/0/0 | 209.165.201.1 | 255.255.255.252 | N/A |
|  | S0/1/0 | 209.165.201.9 | 255.255.255.252 | N/A |
| ISP-2 | S0/0/0 | 209.165.201.17 | 255.255.255.252 | N/A |
|  | S0/1/1 | 209.165.201.13 | 255.255.255.252 | N/A |
| ISP-3 | S0/0/0 | 209.165.201.21 | 255.255.255.252 | N/A |
|  | S0/1/0 | 209.165.201.10 | 255.255.255.252 | N/A |
|  | S0/1/1 | 209.165.201.14 | 255.255.255.252 | N/A |
| REMOTE | S0/0/0 | 209.165.201.2 | 255.255.255.252 | N/A |
|  | G0/0 | 192.168.20.1 | 255.255.255.0 | N/A |
|  | Tunnel 10 | 10.1.1.1 | 255.255.255.252 | N/A |
| HQ | S0/0/0 | 209.165.201.18 | 255.255.255.252 | N/A |
|  | G0/0 | 192.168.30.1 | 255.255.255.0 | N/A |
|  | Tunnel 10 | 10.1.1.2 | 255.255.255.252 | N/A |
| BRANCH | S0/0/0 | 209.165.201.22 | 255.255.255.252 | N/A |
|  | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
| PC1 | NIC | DHCP |  | 192.168.10.1 |
| PC2 | NIC | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| PC3 | NIC | DHCP |  | 192.168.30.1 |
| DNS Server | NIC | 192.168.30.250 | 255.255.255.0 | 192.168.30.1 |

Background / Scenario

In this skills integration challenge, the XYZ Corporation uses a combination of eBGP, PPP, and GRE WAN connections. Other technologies include DHCP, default routing, OSPF for IPv4, and SSH configurations.

Requirements

Note: The user EXEC password is cisco and the privileged EXEC password is class.

Interface Addressing

Configure interface addressing as needed on appropriate devices.

Use the topology table to implement addressing on routers REMOTE, HQ, and BRANCH.

Configure PC1 and PC3 to use DHCP.

SSH

Configure HQ to use SSH for remote access.

Set the modulus to 2048. The domain name is CISCO.com.

The username is admin and the password is secureaccess.

Only SSH should be allowed on the VTY lines.

Modify the SSH defaults: version 2; 60-second timeout; two retries.

PPP

Configure the WAN link from BRANCH to the ISP-3 router using PPP encapsulation and CHAP authentication.

Create a user ISP-3 with the password of cisco.

Configure the WAN link from HQ to the ISP-2 router using PPP encapsulation and CHAP authentication.

Create a user ISP-2 with the password of cisco.

DHCP

On BRANCH, configure a DHCP pool for the BRANCH LAN using the following requirements:

Exclude the first 5 IP addresses in the range.

The case-sensitive pool name is LAN.

Include the DNS server attached to the HQ LAN as part of the DHCP configuration.

Configure PC1 to use DHCP.

On HQ, configure a DHCP pool for the HQ LAN using the following requirements:

Exclude the first 10 IP addresses in the range.

The case-sensitive pool name is LAN.

Include the DNS server attached to the HQ LAN as part of the DHCP configuration.

Configure PC3 to use DHCP.

Default Routing

Configure REMOTE with a default route to the ISP-1 router. Use the Next-Hop IP as an argument.

eBGP Routing

Configure BRANCH with eBGP routing.

Configure BRANCH to peer with ISP-3.

Add BRANCH’s internal network to BGP

Configure HQ with eBGP routing.

Configure HQ to peer with ISP-2.

Add HQ’s internal network to BGP.

GRE Tunneling

Configure REMOTE with a tunnel interface to send IP traffic over GRE to HQ.

Configure Tunnel 10 with appropriate addressing information.

Configure the tunnel source with the local exit interface.

Configure the tunnel destination with the appropriate endpoint IP address.

Configure HQ with a tunnel interface to send IP traffic over GRE to REMOTE.

Configure Tunnel 10 with appropriate addressing information.

Configure the tunnel source with the local exit interface.

Configure the tunnel destination with the appropriate endpoint IP address.

OSPF Routing

Because the REMOTE LAN should have connectivity to the HQ LAN, configure OSPF across the GRE tunnel.

Configure OSPF process 100 on the REMOTE router.

REMOTE should advertise the LAN network via OSPF.

REMOTE should be configured to form an adjacency with HQ over the GRE tunnel.

Disable OSPF updates on appropriate interfaces.

Because the HQ LAN should have connectivity to the REMOTE LAN, configure OSPF across the GRE tunnel.

Configure OSPF process 100 on the HQ router.

HQ should advertise the LAN network via OSPF.

HQ should be configured to form an adjacency with REMOTE over the GRE tunnel.

Disable OSPF updates on appropriate interfaces.

Connectivity

Verify full connectivity from PC2 to the DNS Server.

Verify full connectivity from PC1 to the DNS Server.