Basic rules of  the preparation od cavities:

- Access to the cavity (standard diamons, high speed handpiece)

- Outlines (prevention of extension in case of amalgam)

- Excavation of carious dentin (round bur or excavator, sloe speed)

- Retention (it means that the filling nmust not fall out)

- Resistance (the filling as well as the remaining tooth structure must  not breake)

- Finishing of the borders (we prepare the retentive border for compoiste materials, no retentive bprder for glassionomers and amalgam, the red coded diamond bzr shloud be used.

- Final control – after washing and drying, good illumination.

Diamonds:

Black :extra coarse,

Green :coarse

Blue: standard

Red: fine

Yellow: extra fine

White: ultra fine

Handpieces:

Turbine. 300.000 rpm. For úpreparation of the outlines esp. in enamel, removal of old fillings

Micromotor: max 40.000 rpm. The handpiece can gear the rpm:

Blue :  1:1 (no to slow, no to fast)

Red:    1:5 to fast

Green: 1:1,7 or . 2,5 to slow.

Principle of the connection between the filing material and hard dental tissue:

Composite: micromechanical

Glassionomers: chemical

Amalgam: macromechanical